|  | Test Script SAP S/4HANA Cloud Public Edition - 19-06-26 |
| --- | --- |
|  | Quality Management in Discrete Manufacturing – Consumer Products (5HK_AE) |
|  | PUBLIC |

Table of Contents

1	Purpose	4

2	Prerequisites	5

2.1	System Access	5

2.2	Roles	5

2.3	Master Data, Organizational Data, and Other Data	6

2.4	Business Conditions	10

2.5	Preliminary Steps	11

2.5.1	Attach Document to Inspection Method (Optional)	11

2.5.2	Adopt Routings for Packed Roasted Coffee Beans	12

2.5.2.1	Routings for Packed Medium Roasted Coffee Beans	12

2.5.2.2	Routings for Packed Dark Roasted Coffee Beans	15

2.5.3	Change Inspection Plan for Finished Materials	16

2.5.4	Create Initial Material Stock	18

3	Overview Table	20

4	Test Procedures	25

4.1	Anonymous Forecast and MRP	25

4.1.1	Option 1: Create Planned Independent Requirements	25

4.1.2	Option 2: Create Planned Independent Requirements by Upload File	27

4.1.3	Material Requirements Planning at Plant Level	29

4.1.4	Evaluate Stock/Requirements Situation	31

4.2	Production Order Conversion and Available-to-Promise Check for Packed Roasted Coffee Beans	34

4.2.1	Convert Planned Orders to Production Orders	34

4.2.2	Review Material Availability Status for Production Orders	36

4.3	Production Order Processing for Packed Roasted Coffee Beans	38

4.3.1	Execute Material Staging	38

4.3.2	Release Production Order	39

4.3.3	Goods Issue of Batch-Managed Raw Materials via Pick List	41

4.4	Quality Management – Overview Inspection Lots for Packed Roasted Coffee Beans	42

4.4.1	Display Open Inspection Lots	42

4.5	Quality Inspection – Within Production (Roasting) for Packed Roasted Coffee Beans	44

4.5.1	Display Routing for Inspection Characteristics	44

4.5.2	Display Quality Technician Overview (Optional)	46

4.5.3	Display Open Inspection Lots (Optional)	46

4.5.4	Execute Quality Inspection with Inspection Points for Operation Roasting	48

4.5.5	Display Inspection Results History (Optional)	50

4.6	Production Order Confirmation of Roasting and Cooling Process for Packed Roasted Coffee Beans	51

4.6.1	Confirm Production for Operation Roasting	51

4.6.2	Confirm Production for Operation Cooling	52

4.6.3	Monitor Order Progress (Optional)	54

4.7	Quality Inspection – Early Goods Receipt Inspection/End-Product-Control (Before Goods Receipt)	55

4.7.1	Display Inspection Plan for Inspection Characteristics	55

4.7.2	Display Quality Technician Overview (Optional)	56

4.7.3	Display Open Inspection Lots (Optional)	57

4.7.4	Record Inspection Results (Option 1: Before Goods Receipt)	59

4.7.5	Display Inspection Results History (Optional)	61

4.8	Quality Inspection – In-Process Control (Operation Packaging) and Usage Decision for Packed Roasted Coffee Beans	62

4.8.1	Record Inspection Results (Hierarchical Worklist)	62

4.8.2	Display Inspection Results History (Optional)	65

4.8.3	Display Quality Engineer Overview (Optional)	66

4.8.4	Make Usage Decision	67

4.9	Production Order Confirmation for Packaging Process of Packed Roasted Coffee Beans	70

4.9.1	Confirm Production incl. Component-Backflush for Operation Packaging	70

4.9.2	Post Goods Receipt for Production Order	71

4.9.3	Evaluate Stock/Requirements Situation after Goods Receipt for Production Order	72

4.10	Quality Inspection – Early Goods Receipt Inspection/ End-Product-Control and Usage Decision (After Goods Receipt)	74

4.10.1	Display Material Stock Before Usage Decision	74

4.10.2	Display Open Inspection Lots	75

4.10.3	Record Inspection Results (Option 2: After Goods Receipt)	76

4.10.4	Display Quality Engineer Overview (Optional)	77

4.10.5	Make Usage Decision	78

4.10.6	Display Material Stock after Usage Decision	80

4.11	Production Order Processing and Quality Inspection for Packed Roasted Coffee Beans (Dark)	81

4.11.1	Option 1: Create Initial Material Stock for Packed Roasted Coffee Beans (Dark)	81

4.11.2	Option 2: Execute Process Steps described for Packed Roasted Coffee Beans (Medium) also for Packed Dark Roasted Coffee Beans (Dark)	83

4.12	Production Order Conversion for Coffee Display	85

4.12.1	Create and Release Production Order	85

4.13	Production Order Processing of Coffee Display	87

4.13.1	Goods Issue of Batch-Managed Components	87

4.13.1.1	Option 1: Goods Issue of Batch-Managed Components via Pick List	87

4.13.1.2	Option 2: Manual Goods Issue (Instead of Picking List)	88

4.13.2	Review Production Order (Optional)	89

4.13.3	Confirm Production Order and Post Goods Receipt	91

4.14	Quality Inspection – Goods Receipt Inspection and Usage Decision for Coffee Display	93

4.14.1	Display Material Stock Before Quality Inspection (Optional)	93

4.14.2	Display Inspection Plan for Inspection Characteristics	94

4.14.3	Display Quality Technician Overview (Optional)	95

4.14.4	Display Open Inspection Lots	96

4.14.5	Record Inspection Results (Hierarchical Worklist)	97

4.14.6	Display Quality Engineer Overview (Optional)	99

4.14.7	Make Usage Decision	100

4.14.8	Display Material Stock After Usage Decision	102

4.15	Evaluations	103

4.15.1	Review Scrap Report (Optional)	103

4.15.2	Batch Information Cockpit - Top-Down and Bottom-Up Analysis	104

5	Appendix	106

5.1	Process Integration	106

5.2	Preceding Processes	106

5.3	Succeeding Processes	106

# Purpose

Overview

This scope item outlines a Consumer Products Industry-flavored production process that fully integrates Quality Management into discrete Manufacturing. It operates as a Make-to-stock scenario for batch-managed goods, incorporating QM for In-Process Control and early goods receipt inspection.

The Consumer Industry process involves a two-stage production process for coffee displays. The first stage covers the roasting and packaging of medium respective dark roasted beans, considering the fixed capacity of the roasting drums. During the roasting, cooling and packaging steps, quality inspection continuously monitors the process. During roasting, the temperature, browning of the beans and other parameters are constantly checked by QM until all parameters are fulfilled.

Moreover, the early goods receipt inspection functionality handles the End-Product Control of unpacked, semifinished products as flexibly as possible. This means users can start end-product control during the production process or after receiving goods into quality stock.

The second stage involves a display packaging process where two different semifinished products, such as bags of medium and dark roasted coffee beans, are packed into a display carton. In this step, a quality inspection is set up after receiving goods from production into quality inspection stock.

The process begins with creating a demand forecast for finished goods, represented by Planned Independent Requirements (PIRs). You can create these PIRs manually or by using an upload file. Based on the PIRs, Material Requirements Planning (MRP) generates a production plan for finished goods and breaks down the entire bill of material structure. This results in the planning of semifinished component production and raw material demands. Production planners can analyze and manually adjust the planned orders according to the production plan. Raw material demands lead to purchase requisitions, which trigger alternative procurement scenarios referenced in this scope item.

The production process includes converting planned orders to production orders, order release, material staging and picking, confirming order operations or complete orders, and posting goods receipts. Order-based targets and actual costs are created to ensure fully integrated material and value streams.

This document provides a detailed procedure for testing this scope item after solution activation, reflecting the predefined scope of the solution. Each process step, report, or item is covered in its own section, providing the system interactions (test steps) in a table view. Steps that are not in scope of the process but are needed for testing are marked accordingly. Project-specific steps must be added.

Note Values in this test script (decimal notation, date formats, and so on) are presented in U.S. standard notation. If your test system is set up to use a different notation, enter values as appropriate.

# Prerequisites

This section summarizes all the prerequisites for conducting the test in terms of systems, users, master data, organizational data, other test data and business conditions.

## System Access

| System | Details |
| --- | --- |
| System | Accessible via SAP Fiori launchpad. Your system administrator provides you with the URL to access the various apps assigned to your role. |

## Roles

Create business roles using the following business role templates delivered by SAP and assign them to your individual test users.

Alternatively, if available, you can use the following spaces delivered by SAP. You create a space with pages containing predefined essential apps and assign it to the business role. You then assign this business role to your individual users.

For more information, refer to How to Create a Business Role from a Template in the product assistance for SAP S/4HANA Cloud Public Edition.

| Name (Role Template) | ID (Role Template) | Name (Launchpad Space) | ID (Launchpad Space) | Log On |
| --- | --- | --- | --- | --- |
| Production Engineer - Discrete Manufacturing | SAP_BR_PRODN_ENG_DISC | Production Engineering - Discrete Manufacturing | SAP_BR_PRODN_ENG_DISC |  |
| Warehouse Clerk | SAP_BR_WAREHOUSE_CLERK | Inventory Processing | SAP_BR_WAREHOUSE_CLERK |  |
| Production Planner | SAP_BR_PRODN_PLNR | Production Planning | SAP_BR_PRODN_PLNR |  |
| Production Supervisor - Discrete Manufacturing | SAP_BR_PRODN_SUPERVISOR_DISC | Production Management - Discrete Manufacturing | SAP_BR_PRODN_SUPERVISOR_DISC |  |
| Production Operator - Discrete Manufacturing | SAP_BR_PRODN_OPTR_DISC | Production Execution - Discrete Manufacturing | SAP_BR_PRODN_OPTR_DISC |  |
| Inventory Manager | SAP_BR_INVENTORY_MANAGER | Inventory Management | SAP_BR_INVENTORY_MANAGER |  |
| Quality Planner | SAP_BR_QUALITY_PLANNER | Quality Planning | SAP_BR_QUALITY_PLANNER |  |
| Quality Technician | SAP_BR_QUALITY_TECHNICIAN | Quality Inspection | SAP_BR_QUALITY_TECHNICIAN |  |
| Quality Engineer | SAP_BR_QUALITY_ENGINEER | Quality Engineering | SAP_BR_QUALITY_ENGINEER |  |

## Master Data, Organizational Data, and Other Data

The organizational structure and master data of your company have been created in your system during activation. The organizational structure reflects the structure of your company. The master data represents materials, customers, and suppliers, for example, depending on the operational focus of your company.

Caution Update Behavior of Predelivered Demo Data In non-productive environments (such as starter systems, sandbox systems, or partner demo systems), SAP initially deploys demo data for your selected scope. You can use this data to test and evaluate business processes. In a customer system, you can receive new or updated demo data after initial activation with further scope enhancements, provided the existing demo data wasn't changed in an incompatible way. If activation errors occur due to incompatible changes in demo data, you're responsible for resolving them. If you don't resolve the errors within a specific time frame, SAP reserves the right to deactivate demo data in your systems. In this case, you can continue by creating your own demo data. Alternatively, you can request that your systems be reprovisioned if you want to work with predelivered demo data again. In partner systems, you can use the demo data deployed with the initial activation. No additional demo data will be deployed in the system with scope enhancements. You can only create additional demo data manually.

This scope item requires new master data set to be created and the needed master data objects are mentioned below:

Material Master Data

| Data | Sample Value | Description | Other Attributes |
| --- | --- | --- | --- |
| Material | FG0502 | Display Med + Dark 48 * 1kg Arabica | Batch Managed Finished Material, MTS, QM, Manufacturing Date |
| Material | FG0511 | Carton 12 x 1kg Bags Medium Beans Arabica | Batch Managed Finished Material, MTS with Maximum Lot Size, QM |
| Material | FG0512 | Carton 12 x 1kg Bags Dark Beans Arabica | Batch Managed Finished Material, MTS with Maximum Lot Size, QM |
| Material | PK0501 | Coffee Bag "Medium" 1 kg | Batch Managed Packaging Material, MRP and Costing Relevant |
| Material | PK0502 | Coffee Bag "Dark" 1 kg | Batch Managed Packaging Material, MRP and Costing Relevant |
| Material | PK0504 | Label | Batch Managed Packaging Material, MRP and Costing Relevant |
| Material | PK0507 | Carton for 12 x 1kg Bags | Packaging Material, MRP and Costing Relevant |
| Material | PK0508 | Display Carton Coffee Beans 48 x 1 kg | Packaging Material, MRP and Costing Relevant |
| Material | RM0501 | Raw Coffee Beans- Arabica | Batch Managed Raw Material |

Organization Data

| Data | Sample Value | Description | Other Attributes |
| --- | --- | --- | --- |
| Plant | 3310 | Plant 1 AE |  |
| Storage Location | 331A | Std. storage 1 |  |
| Storage Location | 331B | Std. storage 2 |  |
| Storage Location | 331C | Raw mat. stoloc. |  |
| Controlling Area | A000 | Controlling Area A000 |  |
| Cost center | 33101301 | Manufacturing 1 (AE) |  |

Bill of Material Structure

| Material | Level | Material Type | Unit | Characteristics of Material | Component Quantity | Component UOM |
| --- | --- | --- | --- | --- | --- | --- |
| FG0502 | 0 | FERT | PC | Finished Good for MTS processing with Batch Management | 1 | PC |
| FG0511 | 1 | FERT | PC | Finished Material, Batch Managed, QM, MTS | 2 | PC |
| FG0512 | 1 | FERT | PC | Finished Material, Batch Managed, QM, MTS | 2 | PC |
| PK0508 | 1 | VERP | PC | Packaging Material, Display Carton | 1 | PC |
| PK0504 | 1 | VERP | PC | Label | 1 | PC |

| Material | Level | Material Type | Unit | Characteristics of Material | Base / Component Quantity | Base / Component UOM |
| --- | --- | --- | --- | --- | --- | --- |
| FG0511 | 1 | FERT | PC | Finished Material, Batch Managed, QM, MTS | 10 | PC |
| PK0501 | 2 | VERP | PC | Packaging Material, Batch Managed, External Procurement | 120 | PC |
| PK0504 | 2 | VERP | PC | Label | 10 | PC |
| PK0507 | 2 | VERP | PC | Packaging Material | 10 | PC |
| RM0501 | 2 | ROH | PC | Raw Material, Batch Managed, External Procurement | 138 | KG |

| Material | Level | Material Type | Unit | Characteristics of Material | Base / Component Quantity | Base / Component UOM |
| --- | --- | --- | --- | --- | --- | --- |
| FG0512 | 1 | FERT | PC | Finished Material, Batch Managed, QM, MTS | 10 | PC |
| PK0502 | 2 | VERP | PC | Packaging Material, Batch Managed, External Procurement | 120 | PC |
| PK0504 | 2 | VERP | PC | Label | 10 | PC |
| PK0507 | 2 | VERP | PC | Packaging Material | 10 | PC |
| RM0501 | 2 | ROH | PC | Raw Material, Batch Managed, External Procurement | 144 | KG |

Work Center

| Work Center | Plant | Work Center Category | Work Center Description |
| --- | --- | --- | --- |
| DP050001 | 3310 | 0001 | Display Packaging 1 |
| DR050001 | 3310 | 0001 | Drum Roasting Machine 1 |
| PM050001 | 3310 | 0001 | Packaging Machine 1 |
| CS050001 | 3310 | 0001 | Cooling Sieve 1 |
| DR050002 | 3310 | 0001 | Drum Roasting Machine 2 |
| PM050002 | 3310 | 0001 | Packaging Machine 2 |
| CS050002 | 3310 | 0001 | Cooling Sieve 2 |
| DP050002 | 3310 | 0001 | Display Packaging 2 |

Available capacity for work center

This overview shows predefined shifts (Shift Sequence: Capacity Group PP):

| Shift | Start time | End time | Length of break |
| --- | --- | --- | --- |
| YB1, Early Shift | 06:00:00 | 14:00:00 | 00:30:00 |
| YB2, Late Shift | 14:00:00 | 22:00:00 | 00:30:00 |
| YB3, Night Shift | 22:00:00 | 06:00:00 | 00:30:00 |

Quality Master Inspection Characteristics

| Master data | Value | Plant | Master. data details | Comments |
| --- | --- | --- | --- | --- |
| Master Inspection Characteristic | YQM00001 | 3310 | Roast Color (Medium) |  |
| Master Inspection Characteristic | YQM00002 | 3310 | Roast Color (Dark) |  |
| Master Inspection Characteristic | YQM00003 | 3310 | Temperature |  |
| Master Inspection Characteristic | YQM00004 | 3310 | Weight |  |
| Master Inspection Characteristic | YQM00005 | 3310 | Sensory |  |
| Master Inspection Characteristic | YQM00006 | 3310 | Relative Humidity |  |
| Master Inspection Characteristic | YQM00007 | 3310 | Rebrand |  |
| Master Inspection Characteristic | YQM00008 | 3310 | Visual Pack Check |  |

QM Work Center

| Work Center | Plant | Work Center Category | Work Center Description |
| --- | --- | --- | --- |
| QMCP | 3310 | 0001 | QM Check Point |
| QMLABS | 3310 | 0001 | QM Check Point |

For more information on creating your own master data objects, see the following Master Data Scripts (MDS)

Table 1: Master Data Script Reference

| Master Data ID | Description |
| --- | --- |
| BNR | Create Product Master of type "Raw Material" - MDS |
| 3Y1 | Create Product Master of Type "Packaging" - MDS |
| BNT | Create Product Master of type "Finished Good" - MDS |
| BNJ | Create Production Work Center - MDS |
| BNK | Create Material BOM for Production and Sales - MDS |
| BNL | Create Routing - MDS |
| BLD | Create Production Version - MDS |
| BNQ | Create Quality Inspection Plan - MDS |
| BNY | Create Quality Management Attributes for Material/Product Master - MDS |
| 2ZW | Create Batch Management Master Data - MDS |

## Business Conditions

Before this scope item can be tested, the following business condition must be met.

| Scope Item | Business Condition |
| --- | --- |
| BEG - Standard Cost Calculation | You have completed the step described in the Standard Cost Calculation (BEG) test script. |
| BNZ - Create New Open MM Posting Period - MDS | You have completed the step described in the Create New Open MM Posting Period - MDS (BNZ) master data script. Posting Period is up to date. |
| BNY - Create Quality Management Attributes for Material/Product Master - MDS | Inspection type 03 and 04 are already provided for all relevant materials. |
| BNU - Create Costing Run - MDS | Complete all steps in the test script. |

## Preliminary Steps

### Attach Document to Inspection Method (Optional)

Purpose

Optionally, you can attach documents to inspection methods for detailed information while quality inspection process.

Furthermore, you can expand the long text of inspection method for detailed description of differences between medium and dark roasted coffee beans.

Prerequisite

Inspection Methods COLOR_RB Color Check of Roasted Beans and SENS_RB Sensory of Roasted Coffee Beans are already available in the system.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment. |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Quality Planner. | The Start page appears. |  |
| 2 | Access the SAP Fiori App | Choose Change Inspection Method (QS33). | The Change Inspection Method Version: Initial Screen appears. |  |
| 3 | Enter Initial Data | Make the following entries: - Plant: 3310 - Inspect. Method: e.g., SENS_RB Choose Inspection Method. | The Change Inspection Method Version: General Data screen appears. |  |
| 4 | Navigate to General Data | Select Change Long Text button besides Short text Textbox : Long Text: Please check sensory attributes for fragrance, flavor, sweetness, acidity, body and aftertaste. Go back to General Data screen. | The Change Inspection method long text: Language XX screen appears. |  |
| 5 | Choose a Document to Attach | Choose Documents from General Information. |  |  |
| 6 | Create Attachment for Document | Choose Create Document. In the Change Inspection Method Version: General Data dialog box, choose Continue/Enter without entering data. Confirm the file upload message by choosing OK and select a file from your file system via double-click. Choose Transfer to navigate back to the Change Inspection Method Version: General Data screen. | Document is assigned to inspection method. |  |
| 7 | Save Data | To save your data, choose Save. |  |  |
| 8 | Check next inspection method | Repeat steps 1–3 for Inspection Method COLOR_RB. |  |  |
| 9 | Navigate to General Data | Select Change Long Text button besides Short text Textbox and check, if the following long text appears. - Long Text: Color check of roasted beans. Please use color scale of coffee beans (SCAA) |  |  |
| 10 | Create Attachment | Repeat steps 5–7 for Inspection Method COLOR_RB. |  |  |

### Adopt Routings for Packed Roasted Coffee Beans

To run through this business scenario, the routings for FG0511 and FG0512 need to be enhanced with the allocation of components to routing operations other than operation 0010. Moreover relevant QM inspection data must be maintained in the routing headers and operations 0010 and 0030.

Prerequisite

The routings for the semifinished materials FG0511 and FG0512 are already available in the system.

#### Routings for Packed Medium Roasted Coffee Beans

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as Production Engineer - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Change Routing (CA02). | The Change Routing: Initial Screen is displayed. |  |
| 3 | Enter Allocation Data | On the Change Routing: Initial Screen, make the following entries and choose Continue: - Material: FG0511 - Plant: 3310 On the Change Routing: Overview mark the routing with following Group Counter and choose button Allocation: - GrC: 1 On the Routing Change: Material Component Overview, select the following materials and choose button New Assignment. - Material: PK0501, PK0504, PK0507. If the pop up Choose Alternative BOM should appear, select the following BOM and choose OK: - Usage: 1 - Alt BOM: 1 On the New Assignment pop up, make the following entries and choose Continue: - Activity: 0030 | The allocation of component PK0501, PK0504 and PK0507 to operation 0030 is done. |  |
| 4 | Maintain Header Details | Go Back. On the Change Routing: Overview view, choose button Header. Define Inspection Points Identifier: Y01 Free Insp.Point (Last Sample is valid) | The Change Routing: Header Details view is displayed. |  |
| 5 | Enter Operation Data (Overview) | Choose Header. On the Change Routing: Header Details view, choose button Operations. | The Change Routing: Operation Overview is displayed. |  |
| 6 | Enter Operation Data (Details) for operation 0010 | On the Change Routing: Operation Overview mark the following Operation and choose button Detail: - Operation: 0010 | The Change Routing: Operation Details is displayed. |  |
| 7 | Change Operation Data (Details) | On the Change Routing: Operation Details view, scroll to review the Quality Management: Inspection Points section. - Select No Reference - Insp.point Completion: Automatic valuation based on insp. point definition Choose Back to return to the Operation Overview view. | Note The Reference influences the inspection point creation in results recording later.. |  |
| 8 | Enter Operation Data (Details) for operation 0030 | Repeat steps 5 to 7 for the below operation. Operation: 0030 |  |  |
| 9 | Enter Inspection Characteristics for operation 0010 | Mark the operation line 0010 and choose Inspection Characteristics. Assign Master Inspection Characteristic Roast Color (Medium). Master Inspection Characteristic: YQM00001 Sampling Procedure: YFIXFIP1 Assign Master Inspection Characteristic Temperature. Master Inspection Characteristic: YQM00003 Sampling Procedure: YFIXFIP2 Replace Lower and Upper Limit with: 210 - 212 °C . Choose Enter. Choose Back to return to Operations Overview screen. | Master Inspection Characteristics are assigned to the routing. |  |
| 10 | Enter Inspection Characteristics for operation 0030 | Mark the operation line 0030 and choose Inspection Characteristics. Assign Master Inspection Characteristic Weight Master Inspection Characteristic: YQM00004 Sampling Procedure: YFIXFIP3 Replace Lower Limit with: 1050 g Choose Enter. | Master Inspection Characteristics are assigned to the routing. |  |
| 11 | Save Your Data | Choose Save. Note If Check Task List pop-up appears, click Yes to continue.. | Your data is saved. |  |

Repeat steps 1 to 11 for following routing: material/plant combination:

| Material | Plant | GrC | Component to be assigned | Assignment to operation (OpAc) |
| --- | --- | --- | --- | --- |
| FG0511 | 3310 | 2 | PK0501 PK0504 PK0507 | 0030 |

#### Routings for Packed Dark Roasted Coffee Beans

Procedure

Repeat steps 1 to 11 from previous chapter, but with changed data in steps 3 and 9:

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 3 | Enter Allocation Data | On the Change Routing: Initial Screen, make the following entries and choose Continue: - Material: FG0512 - Plant: 3310 On the Change Routing: Overview mark the routing with following Group Counter and choose button Allocation: - GrC: 1 On the Routing Change: Material Component Overview, select the following material and choose button New Assignment. - Material: PK0502 If the pop up Choose Alternative BOM should appear, select the following BOM and choose OK: - Usage: 1 - Alt BOM: 1 On the New Assignment pop up, make the following entries and choose Continue: - OpAc: 0030 | The allocation of component PK0502 to operation 0030 is done. |  |
| 9 | Enter Inspection Characteristics for operation 0010 | Mark the operation line 0010 and choose Inspection Characteristics. Assign Master Inspection Characteristic Roast Color (Dark). Master Inspection Characteristic: YQM00002 Sampling Procedure: YFIXFIP1 Assign Master Inspection Characteristic Temperature. Master Inspection Characteristic: YQM00003 Sampling Procedure: YFIXFIP2 Replace Lower and Upper Limit with: 225 - 230 °C . Choose Enter. | Master Inspection Characteristics are assigned to the routing. |  |

Also repeat steps 1 to 11 from previous chapter with changed data in steps 3 and 9 for following routing: material/plant combination:

| Material | Plant | GrC | Component to be assigned | Assignment to operation (OpAc) |
| --- | --- | --- | --- | --- |
| FG0512 | 3310 | 2 | PK0502 | 0030 |

### Change Inspection Plan for Finished Materials

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori launchpad as a Quality Planner. | The Start page appears. |  |
| 2 | Access the SAP Fiori App | Open Create Inspection Plan (QP01) tile. | Create Inspection Plan (Initial view) appears. |  |
| 3 | Enter Data | In the Create Inspection Plan (Initial view), make the following entries: - Material: FG0511 - Plant: 3310 - Key Date: <today> Choose Continue. | The Create Inspection Plan: Header Details view appears when there is no plan available for the specified material/plant combination. Note If there is a plan available, choose a group from the dialog box and Inspection Plan Create: Task List Overview appears. Choose New Entries to create a new inspection plan.. |  |
| 4 | Maintain Header Details | Create Inspection Plan (Header Details) appears. Enter the following data: - Usage: Choose from the value help list for usage purpose 5 Goods receipt - Status: Choose from the value help list 4 Released (General) - Lot size: Within 1 to 99,999 Choose Operations. | Inspection Plan Create: Operation Overview appears. |  |
| 5 | Enter Operation Data (Overview) | On the Inspection Plan Create: Operation Overview view, enter the following: - Operation Number: 0010 - Work Center: QMLABS - Control Key: QM01 - Description: End-Product-Control Mark the operation line item, then choose Operation. Note If the Operation button is not available, choose Menu > Operation . | Create Inspection Plan: Operation Details appears. Note The Control Key influences the planning and processing of inspection operation, for example whether or not inspection characteristics must be maintained for an inspection operation.. |  |
| 6 | Operation Data (Details) | On the Create Inspection Plan: Operation Details view, scroll to review the Recording View: single values and summarized result (default view) Choose Back to return to the Operation Overview view. |  |  |
| 7 | Enter Inspection Characteristics | Mark the operation line 0010 and choose Inspection Characteristics. Assign Master Inspection Characteristic Sensory Roasted Beans: Master Inspection Characteristic: YQM00005 Sample Procedure: YFIX1 Assign Master Inspection Characteristic Relative Humidity and change the tolerances as follows: Master Inspection Characteristic: YQM00006 Sample Procedure: YFIX2 Replace Upper Limit with: 5% Assign Master Inspection Characteristic Rebrand Master Inspection Characteristic: YQM00007 Sample Procedure: YFIX2 Replace Lower and Upper Limit with: 14 – 17% Choose Enter. | Master Inspection Characteristics are assigned to inspection plan. |  |
| 8 | Save Your Data | Choose Save. | Your data is saved. |  |

Repeat steps 1 to 8 but with changed step 7 for following inspection plan: material/plant combination:

| Material | Plant | GrC |
| --- | --- | --- |
| FG0512 | 3310 | 1 |

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 7 | Enter Inspection Characteristics | Mark the operation line 0010 and choose Inspection Characteristic. Assign Master Inspection Characteristic Sensory Roasted Beans: Master Inspection Characteristic: YQM00005 Sample Procedure: YFIX1 Assign Master Inspection Characteristic Relative Humidity and change the tolerances as follows: Master Inspection Characteristic: YQM00006 Sample Procedure: YFIX2 Replace Upper Limit with: 5% Assign Master Inspection Characteristic Rebrand Master Inspection Characteristic: YQM00007 Sample Procedure: YFIX2 Replace Lower and Upper Limit with: 18 – 20% Choose Enter. | Master Inspection Characteristics are assigned to inspection plan. |  |

### Create Initial Material Stock

Purpose

In a real business scenario, initial material stock are usually purchased from external suppliers through standard purchasing or subcontracting processes. For only testing purpose, this process step shows you how to create and post the initial material stock to storage locations directly.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Warehouse Clerk. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Post Goods Movement (MIGO). | The Post Goods Movement (MIGO) screen displays. |  |
| 3 | Choose Document Type | Make the following entry and choose Enter. - Trans./Event: Goods Receipt - Reference Document for MIGO Transaction: Other - Init.entry of stBal. :561 |  |  |
| 4 | Enter Data | In the Detail Data section, make the following entries and press Enter: On the Where tab: - Goods Movement Type: 561 - Plant: 3310 - Storage Location: 331C for RMXXXX (Raw Materials) materials and PKXXXX (Packaging Materials) On the Quantity tab: - Quantity: <Quantity>, for example, <1000> (for RM0501, PK0501, PK0502, PK0507 and PK0504 add 40000 PC/kg and for PK0508, 5000 PC) On the Material tab: - Material: <BOM Material>, refer to section Master Data, Organizational Data, and Other Data for Bill of Material Structure. You must create initial stock for all materials except for FG0511, FG0512 and FG0502. Ensure that Item OK is selected and choose Next Item to maintain entries for the next material. |  |  |
| 5 | Check | Choose Check. | The document is ready to be posted. |  |
| 6 | Post | Choose Post. | Material document XXX is posted and materials are available in stock. |  |

We recommend that you check stock level for FG0511, FG0512 and FG0502. To do that, log onto the SAP Fiori launchpad as a Production Planner, and open Manage Material Coverage - New Version (F0251A) app. Remove some stock if there is already sufficient amount of stock in store. Otherwise, no planned orders can be generated later. Use Post Goods Movement (MIGO) app with movement type 562 resp. for Init.entry of stBal. 562 without any stock indicator to remove stock and run the test again.

# Overview Table

This scope item consists of several process steps provided in the table below.

If your system administrator has enabled spaces and pages on the SAP Fiori launchpad, the home page will only contain the essential apps for performing the typical tasks of a business role.

You can find all other apps not included on the home page using the search bar.

If you want to personalize the home page and include the hidden apps, navigate to your user profile and choose Settings > App Finder.

| Process Step | Business Role | Transaction/App | Expected Output |
| --- | --- | --- | --- |
| Anonymous Forecast and MRP |  |  |  |
| Option 1: Create Planned Independent Requirements Manually | Production Planner | Maintain PIRs (F3445) | Planned Independent Requirements for Finished Material are created. |
| Option 2: Create Planned Independent Requirements by Upload File | Production Planner | Maintain PIRs (F3445) | Planned Independent Requirements for Finished Material are created. |
| Material Requirements Planning at Plant Level | Production Planner | Schedule MRP Runs (F1339) | Planned Independent Requirements for Finished Materials and also the components are planned. |
| Evaluate Stock/Requirements Situation | Production Planner | Monitor Material Coverage - Net / Individual Segments (F2101A) | Stock/Requirements situation for the planned materials is analyzed. |
| Production Order Conversion and Available-to-Promise Check for Packed Roasted Coffee Beans |  |  |  |
| Convert Planned Orders to Production Orders | Production Planner | Manage Material Coverage - New Version (F0251A) | Planned Orders for Finished Materials are converted to Production Orders. |
| Review Material Availability Status for Production Orders | Production Supervisor - Discrete Manufacturing | Manage Production Orders (F2336) | Missing parts for the production order are displayed. |
| Production Order Processing for Packed Roasted Coffee Beans |  |  |  |
| Execute Material Staging | Production Operator - Discrete Manufacturing | Stage Materials for Production (MF60) | The components needed for the Finished Material production order are moved from various storage locations to the production storage location. |
| Release Production Order | Production Supervisor - Discrete Manufacturing | Manage Production Orders (F2336) | Production order for Finished Material is released. |
| Goods Issue of Batch-Managed Raw Materials via Pick List | Production Operator - Discrete Manufacturing | Pick Components for Production Orders (CO27) | Goods movements are posted. |
| Quality Management – Overview Inspection Lots for Packed Roasted Coffee Beans |  |  |  |
| Display Open Inspection Lots | Quality Technician | Manage Inspection Lots (F2343) | Inspection lots after release of production order are reviewed. |
| Quality Inspection – Within Production (Roasting) for Packed Roasted Coffee Beans |  |  |  |
| Display Routing for Inspection Characteristics | Production Engineer - Discrete Manufacturing | Display Routing (CA03) | The routing setup is reviewed. |
| Display Quality Technician Overview (Optional) | Quality Technician | Quality Technician Overview (F2361) | An overview of open inspection lots is given. |
| Display Open Inspection Lots (Optional) | Quality Technician | Manage Inspection Lots (F2343) | The open inspection lots are checked. |
| Execute Quality Inspection With Inspection Points for Operation Roasting | Quality Technician | Record Results for Inspection Points (F2689) | Quality Inspection with Inspection Points is executed. |
| Display Inspection Results History (Optional) | Quality Technician | Display Results History (F2428) | Inspection results are reviewed. |
| Production Order Confirmation of Roasting and Cooling Process for Packed Roasted Coffee Beans |  |  |  |
| Confirm Production for Operation Roasting | Production Operator - Discrete Manufacturing | Confirm Production Operation (F3069) | Production Order Operation 0010 is confirmed. |
| Confirm Production for Operation Cooling | Production Operator - Discrete Manufacturing | Confirm Production Operation (F3069) | Production Order Operation 0020 is confirmed. |
| Monitor Order Progress (Optional) | Production Supervisor - Discrete Manufacturing | Manage Production Orders (F2336) | The production order progress is reviewed. |
| Quality Inspection – Early Goods Receipt Inspection/End-Product-Control (Before Goods Receipt) |  |  |  |
| Display Inspection Plan for Inspection Characteristics | Quality Technician | Display Inspection Plan (QP03) | The Inspection Plan is reviewed. |
| Display Quality Technician Overview (Optional) | Quality Technician | Quality Technician Overview (F2361) | An overview of open Inspection Lots is given. |
| Display Open Inspection Lots (Optional) | Quality Technician | Manage Inspection Lots (F2343) | Open Inspection Lots are reviewed. |
| Record Inspection Results (Option 1: Before Goods Receipt) | Quality Technician | Record Inspection Results (F1685A) | Inspection Results are recorded. |
| Display Inspection Results History (Optional) | Quality Technician | Display Results History (F2428) | Inspection Results are reviewed. |
| Quality Inspection – In-Process Control (Operation Packaging) and Usage Decision for Packed Roasted Coffee Beans |  |  |  |
| Record Inspection Results (Hierarchical Worklist) | Quality Technician | Record Inspection Results - Hierarchical Worklist (QE51N) | Inspection results are recorded. |
| Display Inspection Results History (Optional) | Quality Engineer | Display Results History (F2428) | Inspection Results are reviewed. |
| Display Quality Engineer Overview (Optional) | Quality Engineer | Quality Engineer Overview (F2360) | The Inspection lots without Usage Decision are displayed. |
| Make Usage Decision | Quality Engineer | Manage Usage Decisions (F2345) | Usage Decision is made. |
| Production Order Confirmation for Packaging Process of Packed Roasted Coffee Beans |  |  |  |
| Confirm Production incl. Component Backflush for Operation Packaging | Production Operator - Discrete Manufacturing | Confirm Production Operation (F3069) | Final confirmation of the last operation is carried out. |
| Post Goods Receipt for Production Order | Warehouse Clerk | Post Goods Receipt for Production Order (F3110) | The goods receipt for the production order is posted. |
| Evaluate Stock/Requirements Situation after Goods Receipt for Production Order | Production Planner | Monitor Material Coverage - Net / Individual Segments (F2101A) | The Stock/Requirements situation after goods receipt is analyzed. |
| Quality Inspection – Early Goods Receipt Inspection/ End-Product-Control and Usage Decision (After Goods Receipt) |  |  |  |
| Display Material Stock before Usage Decision | Warehouse Clerk | Stock - Single Material (F1076) | Stock situation is reviewed. |
| Display Open Inspection Lots | Quality Technician | Manage Inspection Lots (F2343) | The Inspection Lots are reviewed. |
| Record Inspection Results (Option 2: After Goods Receipt) | Quality Technician | Record Inspection Results (F1685A) | Inspection Results are recorded. |
| Display Quality Engineer Overview (Optional) | Quality Engineer | Quality Engineer Overview (F2360) | The Inspection lots without Usage Decision are displayed. |
| Make Usage Decision | Quality Engineer | Manage Usage Decisions (F2345) | Usage Decision is executed. |
| Display Material Stock after Usage Decision | Warehouse Clerk | Stock - Single Material (F1076) | Stock situation is reviewed. |
| Production Order Processing and Quality Inspection for Packed Roasted Coffee Beans (Dark) |  |  |  |
| Option 1: Create Initial Material Stock for Finished Material Packed Roasted Coffee Beans (Dark) | Warehouse Clerk | Post Goods Movement (MIGO) | Stock for Finished Material is created. |
| Option 2: Execute Process Steps described for Packed Roasted Coffee Beans (Medium) also for Packed Dark Roasted Coffee Beans (Dark) | various | various | Finished Material is produced and stock is available. |
| Production Order Conversion for Coffee Displays |  |  |  |
| Create and Release Production Order | Production Planner | Convert Planned Orders - to Production Orders (CO41) | Production Order for Finished Material is created and released. |
| Production Order Processing for Coffee Displays |  |  |  |
| Goods Issue of Batch-Managed Components - Option 1: Goods Issue of Batch-Managed Components via Pick List | Production Operator - Discrete Manufacturing | Pick Components for Production Orders (CO27) | Goods Issue of components is posted. |
| Goods Issue of Batch-Managed Components - Option 2: Manual Goods Issue (Instead of Picking List) | Warehouse Clerk | Post Goods Movement (MIGO) | Goods Issue of components is posted. |
| Review Production Order (Optional) | Production Supervisor - Discrete Manufacturing | Manage Production Operations (F2335) | The production order is reviewed. |
| Confirm Production Order and Post Goods Receipt | Production Operator - Discrete Manufacturing | Confirm Production Order (CO15) | Final confirmation of production order and goods receipt of finished material is posted. |
| Quality Inspection – Goods Receipt Inspection and Usage Decision for Coffee Display |  |  |  |
| Display Material Stock before Quality Inspection (Optional) | Warehouse Clerk | Stock - Single Material (F1076) | Stock Situation is reviewed. |
| Display Inspection Plan for Inspection Characteristics | Quality Technician | Display Inspection Plan (QP03) | The inspection plan setup is reviewed. |
| Display Quality Technician Overview (Optional) | Quality Technician | Quality Technician Overview (F2361) | An overview of open inspection lots is given. |
| Display Open Inspection Lots | Quality Technician | Manage Inspection Lots (F2343) | Open Inspection Lots are checked. |
| Record Inspection Results (Hierarchical Worklist) | Quality Technician | Record Inspection Results - Hierarchical Worklist (QE51N) | Inspection results are recorded. |
| Display Quality Engineer Overview (Optional) | Quality Engineer | Quality Engineer Overview (F2360) | Inspection results without Usage Decision are reviewed. |
| Make Usage Decision | Quality Engineer | Manage Usage Decisions (F2345) | Usage Decision is executed. |
| Display Material Stock After Usage Decision | Warehouse Clerk | Stock - Single Material (F1076) | Stock Situation is reviewed |
| Evaluations |  |  |  |
| Review Scrap Report (Optional) | Production Supervisor - Discrete Manufacturing | Material Scrap - Maximum Deviation (F2035) | The scrap analysis is done. |
| Batch Information Cockpit - Top Down and Bottom-Up Analysis | Quality Technician | Batch Information Cockpit (BMBC) | The batch traceability is executed. |

# Test Procedures

This section describes test procedures for each process step that belongs to this scope item.

## Anonymous Forecast and MRP

### Option 1: Create Planned Independent Requirements

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to create Planned Independent Requirements (PIRs). PIRs are used to perform demand management functions. A planned independent requirement contains one planned quantity and one date, or a number of planned independent requirements schedule lines, such as one planned quantity split according to dates. In Option 1 these PIRs will be loaded by manually.

Note Instead of creating a single requirement, sometimes one or more planned independent requirements can be maintained and included in a requirement plan for mass processing. In this case, the requirements are grouped and maintained under a requirement plan number.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Planner. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Maintain PIRs (F3445). | The Maintain PIRs screen displays. |  |
| 3 | Check Default Setting | Choose the user icon, and select App Settings. In the MRP Settings dialog box, choose Area of Responsibility. On My Area of Responsibility screen, check if the below entry is assigned: Plant 1 AE (3310) MRP Controller 001 (001) Choose AOR Status if the status for the above entry is not yet assigned. Choose AOR Status to unassign the rest of other entries. |  |  |
| 4 | Select Material Items | On the Maintain PIRs screen, make the following entries and choose Go: - Plant: 3310 - Period Indicator: Weekly(W) - Version Active: Select all options - Material: FG0502 | The searched material item displays. If the material filter is not directly available choose Adapt Filters, select the Material Filter, enter the desired material and choose OK. Note If no PIRs for FG0502 have been maintained in the system before, the selection is empty in the app. In that case please leave this app and access the app Create PIRs (MD61), make the following entries on the Create Planned Independent Requirements: Initial Screen and choose Continue: - Plant: 3310 - Period Indicator: Weekly (W) - Version: 00 - Material: FG0502 Choose Save and restart with step 1.. |  |
| 5 | Edit PIRs . | Select the material item and choose Edit in the upper right corner of the screen. On the Edit PIRs screen, enter quantities per week, for example: - Week 1 (current week + 1 week): 40 PC - Week 2 (current week + 2 weeks): 80 PC - Week 3 (current week + 3 weeks): 80 PC - Week 4 (current week + 4 weeks): 100 PC - Week 5 (current week + 5 weeks): 80 PC - Week 6 (current week + 6 weeks): 80 PC - Version is Active: YES | Note If already stock for FG0502 is available, increase the quantity for PIR of current week + 1 for the stock already available. As an alternative, remove the stock to get planned orders in next step.. |  |
| 6 | Save PIR's Draft | Choose Save in the bottom right corner of the screen. | The PIRs draft is saved. |  |

### Option 2: Create Planned Independent Requirements by Upload File

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to use planned independent requirements to perform demand management functions. A planned independent requirement contains one planned quantity and one date, or a number of planned independent requirements schedule lines, such as one planned quantity split over time according to dates. In Option 2, these PIRs will be loaded by Upload File.

Note Instead of creating a single requirement, sometimes a requirements plan that includes one or more planned independent requirements can be maintained for mass processing. In this case, the requirements are grouped and maintained under a requirement plan number.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Planner. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Maintain PIRs (F3445). | The Maintain PIRs screen displays. |  |
| 3 | Check Default Setting | Choose the user icon, and select App Settings. In the MRP Settings dialog box, choose Area of Responsibility. On My Area of Responsibility screen, check if the below entry is assigned: Plant 1 AE (3310) MRP Controller 001 (001) Choose AOR Status if the status for the above entry is not yet assigned. Choose AOR Status to unassign the rest of other entries. Then choose Back. Please note that you need to maintain this AOR Status correctly when you go through the whole test script. |  |  |
| 4 | Select Material Items | On the Maintain PIRs screen, make the following entries: - Plant: 3310 - Period Indicator: Weekly(W) - Version Active: Select all options - Material: FG0502 | Note If no PIRs for FG0502 have been maintained in the system before, the selection is empty in the app. In that case please leave this app and access the app Create PIRs (MD61), make the following entries on the Create Planned Independent Requirements: Initial screen and choose Continue: - Plant: 3310 - Period Indicator: Weekly (W) - Version Active: 00 - Search: FG0502 Choose Save and restart with step 4.. |  |
| 5 | Filter Result | Choose Go to execute. | Material item displays. |  |
| 6 | Download Template | Select the material item and choose Edit in the upper right. Choose button Download to Spreadsheet to download the template to your local work space. Then choose Back without saving. |  |  |
| 7 | Edit PIRs . | Open the Spreadsheet in an adequate program and enter quantities per period, for example: In the upcoming several weeks, enter following values for PIR: - Week 1 (current week + 2 weeks): 40 PC - Week 2 (current week + 3 weeks): 80 PC - Week 3 (current week + 4 weeks): 80 PC - Week 4 (current week + 5 weeks): 100 PC - Week 5 (current week + 6 weeks): 80 PC - Week 6 (current week + 7 weeks): 80 PC - Version is Active: YES Save your PIRs in the spreadsheet. | Note We recommend to check the stock for finished material before entering the PIRs (see prerequisites). If there is still stock available for FG0502, enhance the first PIR for the stock already available to generate a reproducable capacity situation (e.g. actual stock plus 250 PC). To check the stock, you can Log on to the SAP Fiori Launchpad as a Production Planner and Open the app Manage Material Coverage - New Version (F0251A).. |  |
| 8 | Upload PIRs. | On the Maintain PIRs screen, choose button Upload and Edit. On the Pop Up select the file you adjusted in previous step and choose Continue. Review the uploaded data. |  |  |
| 9 | Save PIRs Draft | Choose Save at bottom right. | The PIRs are saved |  |

### Material Requirements Planning at Plant Level

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to use Material Requirement Planning (MRP) to tailor available capacities and receipts on time to suit requirement quantities. As a result, single-item multi-level requirement planning is performed for plant 3310.

Prerequisite

The Make-to-Stock finished goods are planned at plant level. There is now a requirement for FG0502 in plant 3310.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Planner. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Schedule MRP Runs (F1339). | The Application Jobs screen displays. |  |
| 3 | Create New Job | Choose Create. On the New Job screen, make the following entries: For 1. Template Selection section: - Job Template: Material Requirements Planning (MRP) - Job Name: MRP for FG0502 Choose Step 2. For 2. Scheduling Options section: - Start Immediately: <select> Choose Define Recurrence Pattern. On the Scheduling Information screen, make the following entries: - Start Immediately: <select> - Recurrence Pattern: Single Run Choose OK. Choose Step 3. For 3. Parameters section: - Plant: 3310 - Material: FG0502 - Changed BOM Components: <select> - Stock Transfer Materials: <select> - Scheduling: 2 - Planning Mode: 1 Choose Check at the bottom right. Choose Schedule. | A message appears: You can go ahead and schedule the job. |  |
| 4 | Refresh Application Jobs List | To check the job’s status, enter MRP for FG0502 in the search box and choose Go at the top right section of the screen. | The new job is created and is displayed in the Application Jobs table when refreshed. |  |

### Evaluate Stock/Requirements Situation

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to display the stock/requirements situation for Make-to-Stock finished goods (FG0502) and the components relevant for production (finished, packaging and raw materials) in the stock/requirements list after the requirement planning is performed.

Prerequisite

Requirement planning is carried out.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Planner. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Monitor Material Coverage - Net / Individual Segments (F2101A). | The Monitor Material Coverage - Net / Individual Segments (F2101A) screen displays. |  |
| 3 | Check Default Setting | Choose the user icon and select App Settings. In the MRP Settings dialog box, check Manage Material Coverage - New Version (F0251A) that if toggle button is ON. | If the toggle button is ON, in following steps, it will be navigated to Manage Material Coverage - New Version (F0251A). If the toggle button is OFF, in following steps, it will be navigated to Check Material Coverage - Old Version (F0251). In following steps, we describe the steps with the toggle button is ON. |  |
| 4 | Check Area of Responsibility | Choose the user icon and select App Settings. In the MRP Settings dialog box, choose Area of Responsibility. Check if only this entry displays, and choose OK: Plant 1 AE 3310 MRP Controller (001) Choose + to select this entry if not displayed; choose the Delete icon to remove any other plant entries and then choose OK. |  |  |
| 5 | Select Material | Search in the Material bar, select the following materials and then choose Go. - Material: FG0502 - Material: FG0511 - Material: FG0512 - Material: PK0501 - Material: PK0502 - Material: PK0504 - Material: PK0507 - Material: PK0508 - Material: RM0501 | The Materials screen displays. |  |
| 6 | Review Stock / Requirements List | Select the following materials on the screen and choose Manage Materials. - Material: FG0502 - Material: FG0511 - Material: FG0512 | The detailed stock / requirements list for each material displays. |  |
| 7 | Display Planned Order | From the list of materials on the screen, select the material for which you want to check for the planned order. - Material: FG0502 - Material: FG0511 - Material: FG0512 On the Material Details subscreen, choose PldOrd XXX in the MRP Element column. Note The BOM explosion during the MRP run has generated dependent requirements for the demand-driven planned components. If there is not enough inventory to fulfill the requirements, planned orders and purchase requisitions are created for both inhouse-produced and externally-procured materials.. | The selected planned order displays. Note The planned order is generated only when there is storage for these materials. If you want to display more information of the planned order, choose Document Info at the bottom of the dialog box.. |  |
| 8 | Rescheduling Check (Optional) | On the Stock/Requirements List tab, the Rescheduling column displays the rescheduling status of the following MRP elements. - PurReq - Purchase Orders - Planned Orders - Production Orders - Process Orders If needed, you can choose the following icons in Rescheduling column for corresponding functions: - Reschedule in (bring operation forward if the receipt element lies after the requirement date) - Reschedule out (postpone operation, if the receipt element lies before the requirement date) - Plan Process according to schedule - Cancel Process - Excess Stock - Excess in individual segment |  |  |
| 9 | Display Stock situation of Raw and Packaging Materials | Return to the Manage Material Coverage - Net and Individual Segments screen, select following materials and choose Manage Materials - Material: PK0501 - Material: PK0502 - Material: PK0504 - Material: PK0507 - Material: PK0508 - Material: RM0501 |  |  |
| 10 | Simulation and solving shortage situations (Optional) | If a shortage situation for components exists, the number in the Available column is marked in red and an arrow is visible next to the amount. Click on that arrow and you will be guided to a simulation section. In this section you get provided different solutions how to overcome the corresponding shortage situation. Choose Simulate in the Actions column to simulate your preferred option. Afterwards, you can either choose Cancel in the Actions column to return to previous planning situation or choose Apply to take over the proposed simulation in the operative Stock/Requirements List. |  |  |

## Production Order Conversion and Available-to-Promise Check for Packed Roasted Coffee Beans

After planning process for the finished material FG0502 (Display consisting of medium and dark roasted Coffee Beans Bags) and purchasing of the relevant components the production process starts with the production of the components for the display. Two finished materials FG0511 (Carton 12 x 1kg Bags Medium Beans Arabica) and FG0512 (Carton 12 x 1kg Bags Dark Beans Arabica) are needed for the production of the finished material FG0502 (Display Med + Dark 48 * 1kg Arabica), that combines these two finished materials into one display. The production process for the finished material FG0511 (Carton 12 x 1kg Bags Medium Beans Arabica) with full integration into Quality Management is described in the next sections.

The production process consists of three operations: Roasting (of the beans), Cooling and Packaging (in 1 kg bags / 12 bags in one carton). In-process quality inspection is relevant for the roasting and packaging process. Moreover, a goods receipt inspection (as early goods receipt inspection) is part of this process.

### Convert Planned Orders to Production Orders

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

Daily MRP run creates planned orders for goods to be produced internally. This process step shows you how to convert the planned orders to production orders for finished materials when reached the planned start date.

Prerequisite

The MRP run has generated planned orders for FG0511 (finished material).

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Planner. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Manage Material Coverage - New Version (F0251A). | The Manage Material Coverage screen displays. |  |
| 3 | Select Material | On the Manage Material Coverage screen, enter the following data and choose Go. - Material: FG0511 - Plant: 3310 - Shortage Definition: MRP Standard Choose the row of the filtered material to check more details. | The Manage Material Coverage screen displays with more details. |  |
| 4 | Find Planned Order | Check the generated planned orders. If no planned orders are generated, it could be because FG0511 has sufficient amount of stock in store. In this case, remove some stock or enhance the amount for PIRs for FG0502 and run MRP again to generate planned orders. |  |  |
| 5 | Add Planned Order Long Text (Optional) | Click on the planned order number (PldOrd XXXX) under column MRP Element. In the pop-up dialog PldOrd XXXX, choose Change Planned Order. On the Change Planned Order screen, choose tab Long Text, enter the text, and choose Save. | The long text maintained here can be later copied to the converted production order. |  |
| 6 | Convert Planned Order to Production Order | Select Convert from Edit dropdown list in the Actions column for a planned order (PldOrd XXX). In the Convert Planned Order XXX dialog box, choose Convert to Production Order, confirm Delivery Date and Quantity. Choose Convert. Choose the Refresh button on the top of the material overview section, scroll to the created production order and note down the production order number for material FG0511. | The production order is created. Production Order Number ___________________. |  |
| 7 | Convert Planned Order to Production Order for Semifinished Goods (Optional) | Open Manage Planned Orders (F4170). | The Manage Planned Orders screen displays. |  |
| 8 | Filter Planned Orders (Optional) | On the Manage Planned Orders screen, enter the following data and choose Go. - Material: FG0511 In the Planned Orders area, you can check the generated planned orders. Select Convert from Edit drop down list in the Actions column for a planned order (PldOrd XXX) . In the Convert Planned Order XXXX dialog box, choose Production Order, confirm Delivery Date and Quantity. Choose Convert. You can also perform mass conversion for production order/process order/purchase requisition by selecting multiple planned orders and choosing Convert to. The system then performs a check to validate if the selected planned orders can be converted to the designated order types. A success dialog box pops up if all planned orders are validated successfully. Choose OK and the message is displayed: Planned order converted successfully. The successful message is displayed at the bottom left. A warning dialog box pops up to prompt the number of failed orders and possible reasons if failed orders exist. Choose Convert to XXX, and the failed messages and successful messages are displayed at the bottom left. | If no planned orders are generated, please first ensure if AOR is maintained correctly for plant 3310 and MRP controller 001. Check the message: Planned Order XXX converted to Production order XXX |  |

### Review Material Availability Status for Production Orders

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to review material availability status for production orders.

Prerequisite

Production orders are created for semifinished material FG0511.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Supervisor - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Manage Production Orders (F2336). |  |  |
| 3 | Check Default Area of Responsibility (Supervisor) | Choose the user icon and select App Settings. In the Area of Responsibility dialog box, choose Production Supervisor. Check if only this entry displays, and choose OK: Plant 1 AE 3310 MTS DI - Valuated (YB1) Choose + to select this entry if not displayed; choose the Delete icon to remove any other plant entries and then choose OK. | The Manage Production Order screen displays. |  |
| 4 | Select Production Order | On the Manage Production Orders (F2336) screen, choose Adapt Filters to display more filter options. In the Adapt Filters dialog box, choose More Filters. Select Material and Production Plant and choose OK. Choose Go. - Status: Created - Material: FG0511 - Production Plant: 3310 | The Material and Production Plant fields are added to the filter bar. The searched orders are displayed. |  |
| 5 | Review Material Availability Status | In the Issues column, check the Component Issue icon to the right of the order to display the missing components. | For FG0511, the expected missing components in storage location 331B are RM0501, PK0501, PK0504 and PK0507. The raw and packaging materials are supplied after the Material Staging for Finished Materials Step. For FG0512, the expected missing components in storage location 331B are RM0501, PK0502, PK0504 and PK0507. The raw and packaging materials are supplied after the Material Staging for Finished Materials Step. Note However, it’s also possible that the components mentioned above are not a missing component. This happens when you have supplied the components already or if an overstaging has happened for previous orders and these quantities are not reserved, yet.. |  |

## Production Order Processing for Packed Roasted Coffee Beans

### Execute Material Staging

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to stage materials for finished material production.

Note If there is sufficient amount of stock available in the production storage location, then no line items are generated. In this case, you can skip this step and proceed onward.

Prerequisite

Stock must be available.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Stage Materials for Production (MF60). | The Material Staging for Planned Orders screen displays. |  |
| 3 | Enter Details | Make the following entries and choose Execute. In the Staging Types section: - SLoc Level: <select> - Plant: 3310 - Selection Horizon for Reqmts: <date must be in the future - after scheduled start date of production order created> On the Production/Process Orders tab: Order: <Production Order Number for Finished Material> | The Pull List: Storage Location Level screen displays. |  |
| 4 | Choose Replenishment Elements | In the Total Reqmts table, select the line items and choose Replenishment Proposal on the top of the screen. Review staged quantities and choose Replenishment Elements. |  |  |
| 5 | Enter Replenishment Storage Location and Stage Material | In the Replen. Element table, in the RepLoc column, enter 331C for staged materials. In the Batch field, select an appropriate batch available in storage location 331C for the corresponding material and press Enter. Select all entries. Choose Stage. |  |  |
| 6 | Save | Save your entries. Missing materials are transferred to the shop floor from the designated storage area. With this step, the shortage of material RM0501, PK0501, PK0504 and PK0507 for the finished production order of FG0511 is supplied. | Materials are staged for production. |  |

### Release Production Order

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to release the order at header level, which releases the entire order and all its operations. The order and the operations receive the REL (released) status.

Prerequisite

The production order created by the MRP controller is assigned a release date in accordance with the scheduling margin key.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Supervisor - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Manage Production Orders (F2336). | The Manage Production Order screen displays. |  |
| 3 | Check Default Area of Responsibility (Supervisor) | Choose the user icon and select App Settings. In the Area of Responsibility dialog box, choose Production Supervisor, check if this entry displays and choose OK: - Plant 1 AE (3310) - MTS DI - Valuated (YB1) Choose + to select this entry if not displayed; choose the Delete icon to remove any other plant entries. Choose OK. |  |  |
| 4 | Select Production Order | On the Manage Production Orders screen, choose Adapt Filters to display more filter options. In the Adapt Filters dialog box, choose More filters. Select Material and Production Plant and choose OK. Choose Go. Enter the following filter criteria, and choose Go: - Status: Created - Material: FG0511 - Production Plant: 3310 | The Material and Production Plant fields are added to filter bar. The searched order displays. |  |
| 5 | Assign Batch Number to Production Order before Order Release | Select the order you want to release and choose Edit. In the Production Order Change header move to the tab Goods Receipt. Either click on the Create button next to the Batch field to create an automatic batch number or enter a batch number in the field Batch manually. Choose Save. Confirm the popup for the creation of a new batch number. | Make a note of the batch number. Batch: __________ Note By adopting the customizing settings, an automatic creation of batch number either during production order creation or production order release is possible.. |  |
| 6 | Execute Release | Select the searched order once again. Choose Release. |  |  |

Result

The production order is released. With release of the production order, an inspection lot is created for material FG0511 that contains quality inspections for operations 0010 (Roasting) and 0030 (Packaging) which are performed within the production process (In-Process Control).

At the same time, an inspection lot for the goods receipt inspection is created. Thus, the early End Product Control for material FG0511 can be started in parallel to the running production process and before goods receipt for production order is posted.

### Goods Issue of Batch-Managed Raw Materials via Pick List

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The withdrawal of the raw materials is used for the order. This withdrawal can be done by using the picking list.

Note If the material is set for backflush (in material master or resource), the withdrawing is done automatically after you confirm the operation or the whole order. Backflushing should not be used for batch-managed components (to ensure that the physically picked and consumed batch is the same as the recorded batch).

Note In this scope item the packaging materials PK0501 and PK0504 are (even though it is batch managed) subject to backflush due to process requirements and hence picking will only be executed for RM0501.

Prerequisites

Ensure that sufficient material is present on stock. If not enough stock is present, execute Create Initial Material Stock in the preliminary steps of this scope item.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori Launchpad displays. |  |
| 2 | Access App | Open Pick Components for Production Orders (CO27). | The Picking List: Initial Screen displays. |  |
| 3 | Enter Restriction and Run | Make the following entries and choose Execute: Profile: 000002 Production Order: <Order ID> (from previous steps) Plant: 3310 | The Order Information System: Detail List of Components screen displays. |  |
| 4 | Picking Raw Materials | Select the relevant reservations for RM0501 and choose the upper left Picking. | The Picking list screen displays. On this screen, a list of all the selected reservations is displayed. |  |
| 5 | Batch Determination | RM0501 is managed by batches. You have two options: - Manually enter the relevant batch number. - Execute a Batch Determination by selecting the line for RM0501 and choose button Batch Determination. The Batch determination result is displayed. You can either do changes in the proposed selection or directly accept the proposal. Choose Copy. The number of line items available now in the Picking list corresponds to the number of selected batches. | Back to Picking list screen. If multiple line items were selected in this step, the Picking list screen is displayed after completion of the last line item. |  |
| 6 | Post | Choose Post to post the goods movements. | The goods movements have been posted. |  |

## Quality Management – Overview Inspection Lots for Packed Roasted Coffee Beans

### Display Open Inspection Lots

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection lots are used to record the results for In-Process Control and the early Goods Receipt Inspection (End-Product Control), In this step, the references to production order are verified as well as the inspection lot quantities, sample sizes and postings can be reviewed.

The early goods receipt inspection can be started after Cooling (operation 0020) and while Packaging (operation 0030). Thus, dependent from capacity in the laboratory, the results recording can be started before goods receipt for production order is done.

Prerequisite

In this scope item, two inspection lots with different inspection lot origins are created automatically upon release of previous production order. Due to material master settings, the goods receipt inspection can be performed early.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Inspection Lots (F2343). | The Manage Inspection Lots (F2343) screen displays. |  |
| 3 | Filter Fields | Make the following entries and choose Go: - Plant: 3310 - Material: FG0511 - Inspection Lot Status: Outstanding | Alternatively, you can search for the batch <Batch ID> (from previous step). |  |
| 4 | Select the Corresponding Inspection Lot | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step). | The Production Order number assigned with Inspection lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Check Inspection Lot | Under Inspection Type, check the different inspection types. Under Batch, check the batch from previous step. | Make a note of the inspection lot IDs: Inspection Lot ______________ with inspection type 03 (In-process insp. for production order). Inspection Lot ______________ with inspection type 04 (Goods receipt inspection from production). |  |
| 6 | Check First Inspection Lot by Selecting the Line | In the Origin tab, verify the referenced production order and inspection lot origin. | The production order number from which the inspection lot originates is displayed. Origin: Production |  |
| 7 | Check Inspection Lot Quantities | In the Origin tab, check the Lot Quantities, Sample Size and Postings. | The quantities according to the released production order quantity are displayed. The calculated sample size is displayed. Posting = 0 (not relevant). |  |
| 8 | Check Second Inspection Lot by Selecting the Line | In the Origin tab, verify the referenced production order and inspection lot origin. | The production order number from which the inspection lot originates is displayed. Origin: Goods Receipt from Production |  |
| 9 | Check Inspection Lot Quantities | In the Origin tab, check the Lot Quantities, Sample Size and Postings. | The quantities according to the released production order quantity are displayed. The calculated sample size is displayed. Posting = 0 (not yet started). |  |

## Quality Inspection – Within Production (Roasting) for Packed Roasted Coffee Beans

### Display Routing for Inspection Characteristics

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

An inspection during production (In-Process Control) is triggered by the release of a production order. When you release the production order, an inspection lot is generated, containing the inspection characteristics for operation 0010 (Roasting) and 0030 (Packaging). This inspection lot offers inspection points for repeating the inspections several times, until the expected results are reached (e.g. roast color of coffee beans). With inspection point Y01 in the routing header, only the last sample is valid for usage decision.

In this step, you display the routing (group counter 1 or 2) that is related to the finished good. The difference between the routings are only different drum roasting lines (Drum Roasting Line 1 and 2) used for production. Regarding quality inspection, it is exactly the same.

The routing defines all inspection characteristics used later in the quality inspection. As well you can review the inspection points definition for operation 0010 and 0030.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Engineer - Discrete Manufacturing. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Routing (CA03). | The Display Routing: Initial Screen displays. |  |
| 3a | Enter Selection Fields | Make the following entries and choose Continue. Material: FG0511 Plant: 3310 | The Display Routing: Overview displays. There are two routings (Drum Roasting Line 1 and Drum Roasting Line 2) with respective work centers in PP. The QM data remains the same. |  |
| 3b | Select Group Counter | Select Group Counter: e.g. 1 and choose Operations | The Display Routing: Operation Overview displays. |  |
| 4 | Review the Inspection Points Setting (operation 0010) | Double-click Operation 0010. Scroll down to the Quality Management: Inspection Points section. After reviewing, choose Back. | A No Reference inspection point is defined for this operation in the routing. Insp. point Completion is: Automatic valuation based on insp. Point definition |  |
| 5 | Review the Inspection Points Setting (operation 0030) | Repeat step 4. Double-click Operation 0030. |  |  |
| 6 | Review the Inspection Characteristics (operation 0010) | Select Operation 0010 and choose Inspection Characteristics. There are two Master Inspection Characteristics: One is Qualitative - Roast Color (medium) with Inspection MethodColor Check of Roasted Beans. The other is Quantitative - Temperature. For both, sample size 1 is to be inspected. | The Display Routing: Characteristics Overview displays. If Inspection Characteristics is not displayed, choose More. In the dialogue box, find Inspection Characteristics. |  |
| 7 | Review the Inspection Characteristics (operation 0030) | Repeat step 6. Select Operation 0030 and choose Inspection Characteristics. There is one Master Inspection Characteristic, which is Quantitative - Weight with 3 samples to be inspected. | You have now reviewed the inspection characteristics for which results have to be maintained within a later step. The sampling procedure can be displayed in detail with app Display Sampling Procedures (F2255) (optional). |  |

### Display Quality Technician Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, you can display an overview of open inspection lots via the overview page for quality technicians.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Technician Overview (F2361). | The Quality Technician Overview (F2361) screen displays. You can get an overview of the inspection lots with open results recording and the inspection lots without an inspection plan. Choose card Inspection Lots with Open Results Recording for Inspection Points if you want to record results for the In-Process Control inspection lot. |  |

### Display Open Inspection Lots (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The In-Process Control inspection lot is used to record the results during production. In this optional step, the operations and characteristics are reviewed as well as assigned inspection methods with attachments used for quality inspection.

Prerequisite

In this scope item, the inspection lot is created automatically upon release of production order.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Inspection Lots (F2343). | The Manage Inspection Lots screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0511 - Inspection Lot Status: Outstanding - Inspection Lot Origin: Production | Alternatively, you can search for the batch <Batch ID> (from previous step). |  |
| 4 | Select the corresponding Inspection Lot | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step) | The Production Order number assigned to Inspection Lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. If needed, press button Show Details to make the column visible. |  |
| 5 | Choose Operation (optional) | In Result, choose operation Roasting (Medium). | Inspection Characteristics Roast Color and Temperature can be reviewed with their specifications and calculated sample sizes. |  |
| 6 | Choose Characteristic (optional) | Click Roast Color in the characteristics column. | For checking the attachment in the inspection method: If you have attached a document in the inspection method as described in the Preliminary Steps, the Quality Technician can review the attachment referring to step 6 to 8. A dialog box appears with the Characteristics Details with the link to the Inspection Method. |  |
| 7 | Display the Attachment (optional) | Under Documents, the attachment for an illustration of the Roast Color of coffee beans is displayed. |  |  |
| 8 | Display Text | In General, Long Text is displayed. |  |  |
| 9 | Choose next Operation (optional) | In Result, choose operation Packaging | Inspection Characteristic Weight can be reviewed with its specification and calculated sample size. |  |

### Execute Quality Inspection with Inspection Points for Operation Roasting

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, the quality technician performs the quality inspection and records the inspection result(s) for operation 0010 (Roasting) in order to decide whether the next operation of the production order can be executed. The result is recorded in the inspection lot according to the defined inspection parameters in the routing. Here, free inspection points are used for result recording. Leveraging inspection points, you can have several inspections and can record multiple sets of characteristic results for a single operation. Each inspection point is valuated separately with a coded valuation.

Using Inspection point identifier Y01, only the last sample results are valid.

Prerequisite

The inspection is processed based on inspection lot. In this scope item, the inspection lot is automatically created upon release of the production order.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access App | Open Record Results for Inspection Points (F2689). | The Record Results for Inspection Points screen displays. |  |
| 3 | Execute Search | If not already shown, expand search header, make the following entries and choose Go: Order: <Order ID> Work Center: DR050001 or DR050002 | Selected Order and Work Centers (Drum Roasting Machine 1 or Drum Roasting Machine 2) are displayed in the result list. |  |
| 4 | Open Inspection Points | Click on the line item of the search result. | A list of inspection points is displayed. If inspection point data have not been captured, continue with step 5. |  |
| 5 | Create New Inspection Point | Click On request to create a new inspection point. | Record Results for Inspection Points screen is displayed. |  |
| 6 | Maintain Inspection Results | Maintain the following inspection results and choose Save. a) Negative Result - Roast Color: 10 – Light brown - Temperature: - Valuation: Rejected (R) | Inspection point data is saved and the system displays the list of inspection points. |  |
| 7 | Create Next Inspection Point and Maintain Inspection Results (Optional) | Create a new inspection point as described in step 5 and maintain inspection results as follows: a) Negative Result - Roast Color: 20 – Moderate Light brown - Temperature: - Valuation: Rejected (R) | Further inspection point with inspection results is captured. |  |
| 8 | Create Last Inspection Point and Maintain Inspection Results | Create a new inspection point as described in step 5 and maintain inspection results as follows: a) Positive Result - Roast Color: 30 – Medium shade of brown - Temperature: - Valuation: Accepted (A) | Last inspection point is recorded. Only last sample is valid. Temperature and Roast Color of coffee beans are reached and roasting process can be finished. |  |

### Display Inspection Results History (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, an overview about the history of inspection result is shown. For each quantitative Inspection Characteristic several statistical values and the development for recorded results are illustrated.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Results History (F2428). | The Results History screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. Aggregation Level: Master Inspection Characteristic |  |  |
| 4 | Select Inspection Characteristic | Select an Inspection Characteristic, for example Temperature. | Information is provided about the selected characteristics for process control, for example: - A diagram displays the development of the recorded results for this characteristic with lower and upper limit. - Statistical Values for Control Chart and Process Capability, for example Mean Value. |  |

## Production Order Confirmation of Roasting and Cooling Process for Packed Roasted Coffee Beans

### Confirm Production for Operation Roasting

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In the Consumer Products Industry, goods receipt and goods issue postings are usually done immediately during the physical movements of the materials to ensure batch traceability and data consistency. Order confirmations with backflush are done either at shift end or day end. The sum of the GI and GR quantity for the given order is then recorded within the confirmation and scrap quantities (optional). The time ticket confirmation allows recording either the default times for machine and labor usage, or record actual times for major deviations.

Prerequisite

The production order for the finished material FG0511 has been released and material staging has been carried out.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Confirm Production Operation (F3069). | The Enter Order and Operation screen displays. |  |
| 3 | Enter Order Number and Operation | On the Enter Order and Operation screen, make the following entries, and choose Go: - Order: <Order ID> (from previous steps) - Operation: 0010 | The Confirm Production Operation (F3069) screen displays. |  |
| 4 | Enter Fields in Quantities Section | In the Quantities section, check the following entries and make changes if required: - Yield: <enter the amount you want to confirm> - Scrap: <enter the amount you want to scrap> |  |  |
| 5 | Check Material Movements | The system shows you an overview table of all materials for which the withdrawal is done automatically with operation 0010 with the confirmation of the operation. Note All materials assigned to operation 0010 are not subject to backflush and the list is empty.. |  |  |
| 6 | Enter Fields in Activities Section | In the Activities section, make the following entries: - Setup: <setup time> - Machine: <machine time> - Labor: <labor time> | Confirmation is made. |  |
| 7 | Save your Entries | Choose Post and Complete to make final confirmation for this operation. | Confirmation has been created. Note For partial confirmation choose Post instead of Post and Complete.. |  |

### Confirm Production for Operation Cooling

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In the Consumer Products Industry, goods receipt and goods issue postings are usually done immediately during the physical movements of the materials to ensure batch traceability and data consistency. Order confirmations with backflush are done at shift end or day end. The sum of the GI and GR quantity for the given order is then recorded within the confirmation and scrap quantities (optional). The time ticket confirmation allows recording either the default times for machine and labor usage, or record actual times for major deviations.

Prerequisite

The production order for the seminfinished material FG0511 has been released and material staging has been carried out. The operation 0010 of the production order has already been confirmed.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Confirm Production Operation (F3069). | The Enter Order and Operation screen displays. |  |
| 3 | Enter Order Number and Operation | On the Enter Order and Operation screen, make the following entries, and choose Go: - Order: <Order ID> (from previous steps) - Operation: 0020 | The Confirm Production Operation (F3069) screen displays. |  |
| 4 | Enter Fields in Quantities Section | In the Quantities section, check the following entries and make changes if required: - Yield: <enter the amount you want to confirm> - Scrap: <enter the amount you want to scrap> |  |  |
| 5 | Check Material Movements | In the Material Movements section, check the material list. Note For operation 0020 no components relevant to backflush are assigned. Hence the list is empty.. | The system shows you an overview table of all materials for which the withdrawing is done automatically with the confirmation of the operation. |  |
| 6 | Enter Fields in Activities Section | In the Activities section, make the following entries: - Machine: <enter the setup time you want to confirm for this operation> |  |  |
| 7 | Save your Entries | Choose Post and Complete to make final confirmation for this operation. | Confirmation has been created. Note For partial confirmation choose Post instead of Post and Complete.. |  |

### Monitor Order Progress (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

After the release or (partial) confirmation of finished goods orders is performed, the production supervisor could display and check the order using the app Manage Production Orders for the finished material FG0511.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Supervisor - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Manage Production Orders (F2336). | The Manage Production Orders (F2336) screen displays. |  |
| 3 | Select Production Order | On the Manage Production Orders (F2336) screen, make the following entries: To check default Area of Responsibilities (supervisor), select the user (from the top left corner), App Settings > Area of Responsibility Production Supervisor. Check only this entry is displayed. - Plant 1 AE (3310) - MTS DI - Valuated (YB1) Click Adapt Filters to display more selection filter. Select the checkboxes for Material and Production Plant and click OK. Material: FG0511 Production Plant: 3310 Click Go to execute. | Selected order displays. |  |
| 4 | Check detailed operations status | Check detailed information for selected production operations. Issues: - Empty, if there is no issue for production order - Delay or Missing Components / PRT issue or Quantity Deviation / Quality Issue information shows if relevant issue exists Components: Material, total quantity and issued / open quantity information for order components is displayed Order Schedule: Detailed operation information with relevant work center, confirmed quantity, start and end time is displayed Work Center Schedule: The relevant order covering the same work center capacity (operation is partially confirmed) is displayed Operations not Started: The relevant order that would cover the same work center capacity (operation is released) is displayed Confirmation: Detailed confirmation information with Yield, Scrap, Rework quantity displays if confirmation executed Inspection: Detailed inspection lots information displays if existed | Detailed operations status displays. |  |

Note To display the inspection lots, you need further authorizations of SAP_BR_QUALITY_TECHNICIAN (transaction QA03). Ensure that your user has the appropriate authorizations if you want to review the inspection lots. Else this section is empty even though inspection lots are available.

## Quality Inspection – Early Goods Receipt Inspection/End-Product-Control (Before Goods Receipt)

### Display Inspection Plan for Inspection Characteristics

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

By the release of the production order, the early goods receipt inspection is triggered as well. While inspection lot creation the relevant inspection plan is assigned to the inspection lot.

In this step, you display the inspection plan that is related to the finished good. The inspection plan defines all inspection characteristics used later in the quality inspection (End-Product Control).

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Inspection Plan (QP03). | The Display Inspection Plan: Initial Screen displays. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Continue. Material: FG0511 Plant: 3310 |  |  |
| 4 | Go to Inspection Characteristics Overview Screen | Select the entry for Operation 0010 and choose Inspection Characteristics. Note If you could not find Inspection Characteristics in your screen, choose Menu > Inspection Characteristics .. | The Display Inspection Plan: Characteristic Overview screen displays. |  |
| 5 | Review the Inspection Characteristics | There are three Master Inspection Characteristics: One is Qualitative – Sensory Roasted Beans with inspection method Sensory of Roasted Coffee Beans. Two others are Quantitative – Relative Humidity and Rebrand. For all, sample size 1 is to be inspected. Auto-defect recording is defined if characteristics are rejected. | You have now reviewed the inspection characteristics for which data has to be maintained later. |  |

### Display Quality Technician Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, you can display an overview of open inspection lots via the overview page for quality technicians.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Technician Overview (F2361). | The Quality Technician Overview (F2361) screen displays. You can get an overview of the inspection lots with open results recording and the inspection lots without an inspection plan. Choose card Inspection Lots with Open Results Recording, No Inspection Points if you want to record results for the early goods receipt inspection lot. |  |

### Display Open Inspection Lots (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The early goods receipt inspection lot is used to start the End-Process Control already before goods receipt. In this optional step, the operations and characteristics are reviewed as well as assigned inspection methods with attachments used for quality inspection.

Prerequisite

In this scope item, the inspection lot is created automatically upon release of production order.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Inspection Lots (F2343). | The Manage Inspection Lots screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0511 - Inspection Lot Status: Outstanding - Inspection Lot Origin: Goods Receipt from Production | Alternatively, you can search for the batch <Batch ID> (from previous step). |  |
| 4 | Select the corresponding Inspection Lot | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step) | The Production Order number assigned to Inspection Lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. If needed, press button Show Details to make the column visible. |  |
| 5 | Choose Operation (optional) | In Result, choose operation End-Product-Control. | Inspection Characteristics Sensory Roasted Beans, Relative Humidity and Rebrand can be reviewed with their specifications and calculated sample sizes. |  |
| 6 | Choose Characteristic (optional) | Click Sensory Roasted Beans in the characteristics column. | For checking the attachment in the inspection method: If you have attached a document in the inspection method as described in the Preliminary Steps, the Quality Technician can review the attachment referring to step 6 to 8. A dialog box appears with the Characteristics Details with the link to the Inspection Method. |  |
| 7 | Display the Attachment (optional) | Under Documents, the attachment for an illustration of the Roast Color of coffee beans is displayed. |  |  |
| 8 | Display Text | In General, Long Text is displayed. |  |  |

### Record Inspection Results (Option 1: Before Goods Receipt)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, the quality technician performs the quality inspection and records the inspection results for operation 0010 (End-Product Control).

After Cooling, the Quality Technician takes a sample of unpacked Roasted Beans and starts with quality inspection. This step can be performed alternatively before goods receipt (Option 1) or after goods receipt (Option 2).

It´s also possible to record results for some MICs earlier (e.g. rebrand and relative humidity), for others later (e.g. sensory), dependent from capacity in the laboratory.

Procedure

Record Inspection Results for one Inspection Lot

If inspection results shall be recorded for one or only a few inspection lots, execute the following steps:

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log Onto SAP Fiori Launchpad | Log onto the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Record Inspection Results (F1685A) | The Results Recording screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. Plant: 3310 Material: FG0511 Order: <Order ID> from previous step. | If Order is not displayed in the Filters, please choose Filters. On the popup screen Filters, select Order and choose OK. Choose Go. |  |
| 4 | Select the Corresponding Inspection Lot | In the inspection lot list, select the inspection lot. Choose Record Multiple Results. |  |  |
| 5 | Enter Inspection Result | Choose Edit Depending on the inspection result, select one of the following two options: Positive Result Sensory: OK Relative Humidity: Mean Value (%): 4,0 Rebrand: Mean Value (%): 14 Negative Result Sensory: NOK Relative Humidity: Mean Value (%): 5,2 Rebrand: Mean Value (%): 18 | For the qualitative characteristic (Sensory) the search help is needed to select a value. For the quantity characteristic (Relative Humidity and Rebrand) a number can be entered directly. The valuation and close are performed automatically, and the status is set to Valuated. The default number of inspected objects is calculated based on the sample size reviewed in an earlier step. The number of inspected objects can be manually changed. As well a number of nonconforming objects can be maintained directly in the respective fields for every inspection characteristic. |  |
| 6 | Save Inspection Lot | Choose Save. | The inspection lot is saved. QM Inspection for finished material has been performed and result recorded. |  |

Optional: Process-Optimized Results Recording for Multiple Inspection Lots

If inspection results shall be recorded for several inspection lots at the same time, execute the following steps.

Caution Note that this app can only be used for quality inspections with master inspection characteristics. It is not possible to capture inspection results for inspections with plan characteristics.

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log Onto SAP Fiori Launchpad | Log onto the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Record Inspection Results in Table Form (F3365) | The Record Inspection Results in Table Form screen displays. |  |
| 3 | Enter Filter Fields | Maintain filter values and choose Go. | A list of inspection lots is displayed. |  |
| 4 | Select Inspection Lots and Inspection Characteristics | Select the inspection lots for which inspection results shall be recorded and choose Record Multiple Results. In the dialogue box Select Characteristics for Result Recording, select the master inspection characteristics for which inspection results shall be recorded and choose button OK. | The Record Inspection Results in Table Form screen displays. Note The system creates one table line item per inspection lot operation. Each master inspection characteristic is displayed in a separate column. The valuation of the inspection characteristic is displayed in a separate adjacent column. If a master inspection characteristic is assigned to another inspection operation, the respective table field is grayed out.. |  |
| 5 | Show Inspection Details (Optional) | Click one inspection characteristic in the table and choose Show Details. | The side panel opens and detailed inspection data are displayed (e.g. inspection specifications, master inspection characteristic, inspection method, test equipment, and inspection remarks). |  |
| 6 | Show Work Center (Optional) | Choose Settings button in the upper right corner of the table. In the Define Colunm Properties dialogue box, select Work Center and choose OK. | An additional Work Center column is displayed. |  |
| 7 | Enter Inspection Results | Enter inspection results. See above for example data. | For qualitative results, a corresponding value help is provided. Quantitative inspection results can be entered directly. Depending on the defined valuation mode (check inspection details), the valuation is either done automatically by the system or must be done manually by the user. If a inspection result requires an inspection remark (check master inspection characteristic settings), it is not possible to navigate to the next inspection result. Open Details side panel and maintain the inspection remark in section Remarks. |  |
| 8 | Save Inspection Data | Choose Save. | Inspection results are saved and a success message is displayed. |  |

### Display Inspection Results History (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, an overview about the history of inspection result is shown. For each quantitative Inspection Characteristic several statistical values and the development for recorded results are illustrated.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Results History (F2428). | The Results History screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. Aggregation Level: Master Inspection Characteristic |  |  |
| 4 | Select Inspection Characteristic | Select an Inspection Characteristic, for example, Relative Humidity and Rebrand. | Information is provided about the selected characteristics for process control, for example: - A diagram displays the development of the recorded results for this characteristic with lower and upper limit. - Statistical Values for Control Chart and Process Capability, for example Mean Value. |  |

## Quality Inspection – In-Process Control (Operation Packaging) and Usage Decision for Packed Roasted Coffee Beans

### Record Inspection Results (Hierarchical Worklist)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, the quality technician performs the quality inspection and records the inspection result(s) for last operation 0030 (Packaging) within the production process.

The result is recorded in the inspection lot according to the defined inspection parameters in the routing. Here, free inspection points are used for result recording. Leveraging inspection points, you can have several inspections and can record multiple sets of characteristic results for a single operation. Each inspection point is valuated separately with a coded valuation.

Using Inspection point identifier Y01, only the last sample results are valid.

Prerequisite

The inspection is processed based on inspection lot. In this scope item, the inspection lot is automatically created upon release of the production order.

Production order operations 0010 (Roasting) and 0020 (Cooling) have already been confirmed.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Record Inspection Results - Hierarchical Worklist (QE51N). | The Results Recording Worklist screen displays. Note The app Record Results for Inspection Points (F2689) can be used as an alternative to capture inspection results for inspection points. However, recording of different single values currently causes an issue. Therefore, use app Record Inspection Results - Hierarchical Worklist (QE51N).. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Execute. Plant: 3310 Insp. Lot Origin: 03 (Production) Material: FG0511 Batch: <Batch ID> from previous step Lot Created: <choose one week ago until today> Work Center: PM050001 or PM050002 | The Results Recording Worklist screen displays. Caution In Cloud system, Dynamic Selection (e.g. for production order) is not offered. Alternatively you can search for the <Batch ID> or <Inspection Lot ID> from previous step.. Selected work centers (Packaging Machine 1 or Packaging Machine 2) are displayed in the result list. |  |
| 4 | Select the Corresponding Inspection Lot | On the left pane, expand the corresponding inspection lot which you want to record a result. | If a dialogue appears asking to save, choose Cancel. |  |
| 5 | Choose the First Inspection Point | Choose the first inspection point Further Inspection which is under the activity 0030 Packaging 1 KG Work Center Packaging Machine with a click. | When the work center is selected, the cursor jumps automatically to the result. |  |
| 6 | Enter Inspection Result for the First Inspection Point | Depending on the inspection result, enter your values: e.g. - Negative Result - Weight – Result: 1048, 1049, 1050 | For the quantitative characteristic Weight, three independent values need to be entered and the system will calculate the mean value. |  |
| 7 | Close and Save the First Inspection Point | Choose Enter. Choose Save. | Auto Valuation Rejected is done by the system. The status is set to 5 Processing is completed. Note If you want to run a rework process for material FG0511 due to failed quality inspection, you can follow the test script 6A9 (Rework with Quality Management – Consumer Products). You need to execute preliminary step “Assign Reference Operation Set and Order Type to Routing” of 6A9 test script and then follow the test steps “Execute Partial Production Order Confirmation for Operation Roasting” until “Analyze Event-based Order Costs“ within section “Rework Production Orders within Production Execution”. Afterwards you continue in this test script with chapter “Production Order Processing and Quality Inspection for Packed Roasted Coffee Beans (Dark)”.. |  |
| 8 | Create the Next Inspection Point | Choose Further Inspection which is under the activity 0030 Packaging 1 KG Work Center Packaging Machine with a click. | The next inspection point is created with current Date/Time. A dialogue appears asking to save data for operation 0030, Choose Yes. |  |
| 9 | Enter Inspection Result for the Next Inspection Point | Depending on the inspection result, enter your values: e.g. - Positive Result - Weight – Result: 1050, 1051, 1052 | For the quantitative characteristic Weight, three independent values need to be entered and the system will calculate the mean value. |  |
| 10 | Close and Save the Next Inspection Point | Choose Enter. Choose Save. | Auto Valuation Accepted is done by the system. The status is set to 5 Processing is completed. |  |

### Display Inspection Results History (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, an overview about the history of inspection result is shown. For each quantitative Inspection Characteristic, several statistical values and the development for recorded results are illustrated.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on | Log on to the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Results History (F2428). | The Results History screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. Aggregation Level: Master Inspection Characteristic |  |  |
| 4 | Select Inspection Characteristic | Select an Inspection Characteristic, for example, Weight. | Information is provided about the selected characteristics for process control, for example: - A diagram displays the development of the recorded results for this characteristic with lower and upper limit. - Statistical Values for Control Chart and Process Capability, for example Mean Value and Variance. |  |

### Display Quality Engineer Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection is done and the inspection result is recorded. You must decide whether or not to accept the inspection lot depending on the inspection result. As an optional entry point for making this usage decision, you can use the app Quality Engineer Overview (F2360). Furthermore, the Quality Engineer Overview (F2360) app displays inspection lots without an inspection plan (e.g. if automatic assignment of an inspection plan is not set up in the material master or if automatic assignment failed), inspection lots without usage decision and inspection lots ready for usage decision.

Caution For this scope item example, auto-defect recording has not been performed in quality inspection within production (In-Process Control) because deviations are expected during the production process and shall not be tracked as defects later.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Engineer Overview (F2360). | The Quality Engineer Overview (F2360) screen displays. |  |
| 3 | In Card: Inspection Lots without Inspection Plan choose one entry (optional) |  | To assign missing data and ensure automatic inspection plan determination for further usage. |  |
| 4a | In Card: Select Inspection Lots Without Usage Decision (Optional) | Choose card Inspection Lots Without Usage Decision. | You can process further as described in Make Usage Decision Chapter. |  |
| 4b | Select the Corresponding Inspection Lot you have created before (optional) | In the inspection lot list, select the inspection lot. | The Manage Usage Decisions screen displays. Alternative entry to chapter Manage Usage Decision. |  |
| 4c | Select one of the Inspection Lot by Results Recording Status (Optional) | Select one of the Inspection Lot by Results Recording Status (Optional). | Alternative entry to chapter Manage Usage Decision. |  |
| 5 | Select Card Inspection Lots Ready for Usage Decision (optional) |  | All inspection lots with positive results get collectively an accepted usage decision (e.g. defaulted by A1) when pressing button Make Quick Usage Decision will be renamed soon. |  |
| 6 | In Card: Inspection Lots with Defects (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined) Select one of the inspection lot origins, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 7 | In Card: Top Defective Materials (optional) | (In case failed inspection results have been recorded and auto-defect recording has been defined) Select one of the materials inspected, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 8 | In Card: Top Defective Codes (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined) Select one of the defect codes recorded, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. |  |  |

### Make Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspections within production (operation Roasting and Packaging) are done and the inspection results are recorded. The Production Operator - Discrete Manufacturing was informed about the inspection results and can confirm last production order operation 0030 Packaging.

After recording of the inspection results by the quality technician, the quality engineer can now make a usage decision.

In order to close the inspection lot, the quality engineer makes a usage decision. As the inspection lot of origin 03 is not stock-relevant, there is no follow-up action assigned to the usage decision code that triggers a goods movement.

Caution In this example, the In-Process Control is performed as long as the relevant process parameters are reached. Thus, the last sample will always be accepted and because with inspection point Y01 only the last sample is valid, the UD can always be made with UD03 A (Accepted). For performing the UD you can either edit the inspection lot after checking the inspection data or you can perform the Quick Usage decision immediately after inspection lot selection.

Procedure: Usage Decision after checking inspection data

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Usage Decisions (F2345). | The Manage Usage Decisions (F2345) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0511 - Inspection Lot Origin: Production - Usage Decision Made: No | Alternatively you can search for the <Order ID> or <Batch ID> from previous steps. If Production Order is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Production and choose More Filters. Select Production Order and choose OK. Choose Go. If Batch is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Material and choose More Filters. Select Batch and choose OK. Choose Go. |  |
| 4 | Click the Corresponding Inspection Lots | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step). | The Production Order number is assigned to Inspection Lot. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Review of Inspection Points | Select the tab Inspection Points, and review the detailed inspection results recorded previously. (From previous steps – Inspection Operations Roasting and Packaging) |  |  |
| 6 | Enter UD code | Choose Edit and select the tab Usage Decision. The UD code you select depends on the Inspection Result. Please select the following option, Positive Result UD code: UD03 A (Accepted) |  |  |
| 7 | Save Usage Decision | Choose Save. | Usage decision has been made for In-process control inspection lot. Following a positive usage decision, you can now confirm the operation Packaging of the production order. |  |

Procedure: Quick Usage Decision

Alternatively, you can perform the Quick Usage decision immediately after inspection lot selection (without inspection data check).

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Usage Decisions (F2345). | The Manage Usage Decisions (F2345) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0511 - Inspection Lot Origin: Production - Usage Decision Made: No |  |  |
| 4 | Select the Corresponding Inspection Lot | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> from previous step. | If the Production Order column does not appear, please choose the Settings button. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Make Quick Usage Decision | Press Button Make Quick Usage Decision | Usage decision with UD code: UD03 A (Accepted) has been made for In-process control inspection lot. Following a positive usage decision, you can now confirm the operation Packaging of the production order. |  |

## Production Order Confirmation for Packaging Process of Packed Roasted Coffee Beans

### Confirm Production incl. Component-Backflush for Operation Packaging

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In the Consumer Products Industry, Goods receipt and Goods Issue postings are usually done immediately during the physical movements of the materials to ensure batch traceability and data consistency. Order confirmations with backflush are done at shift end or day end. The sum of the GI and GR quantity for the given order is then recorded within the confirmation and scrap quantities (optional). The time ticket confirmation allows recording either the default times for machine and labor usage, or record actual times for major deviations.

In this scope item the goods issue of packaging material is confirmed together with the operation confirmation even though the packaging materials are managed in batches due to machine design.

Prerequisite

The production order for the seminfinished material FG0511 has been released and Material Staging has been carried out. The operations 0010 and 0020 of the production order have already been confirmed.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment. |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App. | Open Confirm Production Operation (F3069). | The Enter Order and Operation screen displays. |  |
| 3 | Enter Order Number and Operation. | On the Enter Order and Operation screen, make the following entries, and choose Go: - Order: <Order ID> (from previous steps) - Operation: 0030 | The Confirm Production Operation (F3069) screen displays. |  |
| 4 | Enter Fields in Quantities Section. | In the Quantities section, make the following entries: - Yield: <enter the amount you want to Confirm> - Scrap: <enter the amount you want to Scrap> |  |  |
| 5 | Check Material Movements. | In the Material Movements section, check the material list. Materials PK0501, PK0504 and PK0507 are subject to backflush and partly managed by batches (PK0501 and PK0504). Choose the Edit button on the right side of the material. Enter the relevant batch number and choose Save. | The system shows you an overview table of all materials for which the withdrawing is done automatically with the confirmation of the operation. The materials PK0501, PK0504 and PK0507 are set for backflush (in material master) and assigned to operation 0030 in the routing of FG0511 so that the withdrawing is done automatically after you confirm the operation. |  |
| 6 | Enter Fields in Activities Section. | In the Activities section, make the following entries: - Machine: <enter the setup time you want to confirm for this operation> - Labor: <enter the setup time you want to confirm for this operation> |  |  |
| 7 | Save your Entries. | Choose Post and Complete to make final confirmation for this operation. | Confirmation has been created. Note For partial confirmation, choose Post instead of Post and Complete.. |  |

### Post Goods Receipt for Production Order

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to post the goods receipt for the production order. You may post goods receipt for the total quantity, but if already partial quantities of the products are required for further processing, for example for further production or subsequent deliveries for sales orders, you can also execute partial confirmation during production process.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Warehouse Clerk. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Post Goods Receipt for Production Order (F3110). | The Post Goods Receipt for Production Order (F3110) screen displays. |  |
| 3 | Enter Details | On the Post Goods Receipt for Production Order (F3110) screen, make the following entries and choose Enter: Order: <Order ID> (from previous steps) |  |  |
| 4 | Select the Item | In the Items section, select the item. | The Goods Receipt Item screen displays. |  |
| 5 | Input Production Date | In the Production and Shelf Life Expiration Date section, make the following entries and choose Apply: Production Date: <Current Date> | The Goods Receipt screen displays. |  |
| 6 | Post | Choose Post. | Goods receipt for production order has been done. |  |

### Evaluate Stock/Requirements Situation after Goods Receipt for Production Order

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to display the stock/requirements situation for the finished good (FG0511) in the stock/requirements list after goods receipt for the production order is executed.

Prerequisite

Goods Receipt for the production order of the finished material is executed.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Planner. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Monitor Material Coverage - Net / Individual Segments (F2101A). | The Monitor Material Coverage - Net / Individual Segments (F2101A) screen displays. |  |
| 3 | Check Default Setting | Choose the user icon and select App Settings. In the MRP Settings dialog box, check Manage Material Coverage - New Version (F0251A) that if toggle button is ON. | If the toggle button is ON, in following steps, it will be navigated to Manage Material Coverage - New Version (F0251A). If the toggle button is OFF, in following steps, it will be navigated to Manage Material Coverage - New Version (F0251A). In following steps, we describe the steps with the toggle button is ON. |  |
| 4 | Select Material | Search in the Material bar, select the following materials and then choose Manage Materials: - Material: FG0511 | The Manage Material Coverage screen displays. |  |
| 5 | Review Stock / Requirements List | Select the corresponding material on the screen on the left side. | The detailed stock / requirements list for each material displays. |  |
| 6 | Check Inspection Lot | From the list on the screen, select the material for which you want to check for the inspection lot. - Material: FG0511 On the screen, you can review the Stock / Requirements Situation. Due to the final confirmation of the production order in previous step the production order isn't relevant for disposition anymore and hence removed from the list. However, due to the goods receipt from production, the early goods receipt inspection lot becomes disposition relevant and visible. You can check for the inspection lot number <Inspection Lot ID> noted down in previous steps. | If you want to display more detailed information of the disposition elements, choose Open > Stock/Requirements List and navigate to the corresponding disposition element. |  |

## Quality Inspection – Early Goods Receipt Inspection/ End-Product-Control and Usage Decision (After Goods Receipt)

### Display Material Stock Before Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, a review of finished material stock is done. Due to quality inspection, the material is not available for other processing.

Prerequisite

The goods receipt from production order was posted for the finished material directly to quality inspection stock. This is caused by material master settings for inspection type 04 Goods Receipt Inspection from production.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Inventory Manager. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Stock - Single Material (F1076). | The Stock - Single Material (F1076) screen displays. |  |
| 3 | Enter Material | Make the following entries and choose Enter. FG0511 | Stock for the material is displayed. |  |
| 4 | Check Stock Quantity | Check the Quality Inspection Stock under the plant 3310 and for your batch <Batch ID>. | Stock for the material, batch and storage location is checked. |  |

### Display Open Inspection Lots

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection lot is used to record the results of the End-Product Control. In this step, the references to the production order and material document (after goods receipt) are verified as well as the inspection lot quantities and postings can be reviewed.

Prerequisite

In this scope item, two inspection lots are created automatically upon goods receipt for the production order.

The In-Process Control (inspection within production) is completed. Production order confirmation is done. Goods receipt is posted.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Inspection Lots (F2343). | The Manage Inspection Lots (F2343) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0511 - Inspection Lot Status: Outstanding | Alternatively, you can search for the batch <Batch ID> (from previous step) |  |
| 4 | Select the Corresponding Inspection Lots | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step). | The Production Order number assigned to Inspection Lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Check Inspection Lot | Under Inspection Type, check the open inspection lot. Press button Show details in the top of the list to watch the progress of characteristics. | Inspection Lot (from previous step) with inspection type 04 is displayed. Progress is dependent from results recording performed in the previous step (Option 1: Before goods receipt) |  |
| 6 | Check progress of characteristics | In the Result tab, verify the already completed characteristics as well as the still open ones. |  |  |
| 7 | Check Inspection Lot by Selecting the Line | In the Origin tab, verify the referenced production order and Inspection Lot Origin. With goods receipt the material document is also assigned to the existing inspection lot. Verify the material document. | The production order from which the inspection lot originates is displayed. Origin: Goods Receipt from Production. Material Document (from previous step). |  |
| 8 | Check Inspection Lot Quantities | In the Origin tab, check the Lot Quantities, Sample Size and Postings. | The Lot Quantity and the calculated sample size are displayed as after release of production order. Actual Lot Quantity and Postings have changed according goods receipt quantity from previous step. |  |
| 9 | Check Material Document (Optional) | In the Stock Postings tab, check details of the assigned Material Document, which made the inspection lot disposition relevant. | Movement Type of this material document is 101 GR goods receipt. Quantity is as posted in the previous step. |  |

### Record Inspection Results (Option 2: After Goods Receipt)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

With early goods receipt inspection it´s possible to start the inspection in parallel to the running production process (Option 1), because the inspection lot is created already with release of the production order.

You can also start or complete the inspection after goods receipt (Option 2) in the existing inspection lot, dependent from capacity in the laboratory.

In this step, the quality technician performs the quality inspection and records the inspection result(s) for operation 0010 (End-Product Control).

Procedure

If there are open inspection characteristics, refer to:

Chapter “Quality Inspection – Early Goods Receipt Inspection (Before Goods Receipt)”

- Record Inspection Results (Option 1: Before Goods Receipt)

### Display Quality Engineer Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection is done and the inspection result is recorded. You must decide whether or not to accept the inspection lot depending on the inspection result. As an optional entry point for making this usage decision, you can use the app Quality Engineer Overview (F2360). Furthermore, the Quality Engineer Overview (F2360) app displays inspection lots without an inspection plan (e.g. if automatic assignment of an inspection plan is not set up in the material master or if automatic assignment failed), inspection lots without usage decision and inspection lots ready for usage decision. If negative inspection results are recorded, you can check the defect details in app Manage Defects (F2649) directed from the app Quality Engineer Overview (F2360) via the cards Inspection Lots with Defects, Top Defective Materials, or Top Defects.

Caution For this scope item example, auto-defect recording is performed in quality inspection for goods receipt (End-Product Control) because deviations shall be tracked as defects later.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Engineer Overview (F2360). | The Quality Engineer Overview (F2360) screen displays. |  |
| 3 | In Card: Inspection Lots without Inspection Plan choose one entry (optional) |  | Assign missing data and ensure automatic inspection plan determination for further usage. |  |
| 4a | In Card: Select Inspection Lots Without Usage Decision (Optional) | Choose card Inspection Lots Without Usage Decision. | You can process further as described in Make Usage Decision. |  |
| 4b | Select the Corresponding Inspection Lot you have created before (optional) | In the inspection lot list, select the inspection lot. | The Usage Decisions screen displays. Alternative entry to Make Usage Decision. |  |
| 4c | Select one of the Inspection Lot by Results Recording Status (Optional) | Select one of the Inspection Lot by Results Recording Status (Optional). | Alternative entry to Make Usage Decision. |  |
| 5 | Select Card Inspection Lots Ready for Usage Decision (optional) |  | All inspection lots with positive results get collectively an accepted usage decision (e.g. defaulted by A1) when pressing button Make Quick Usage Decision. |  |
| 6 | In Card: Inspection Lots with Defects (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the inspection lot origins, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 7 | In Card: Top Defective Materials (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the materials inspected, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 8 | In Card: Top Defects (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the defect codes recorded, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. |  |  |

### Make Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection is done and the inspection result is recorded. You must decide whether or not to accept the finished material depending on the inspection result. If you do not want to use the Quality Engineer Overview(F2360), you can open the Manage Usage Decisions(F2345) app directly.

In order to close the inspection lot, the quality engineer makes a usage decision. As the inspection lot of origin 04 is stock-relevant, there is a follow-up action assigned to the usage decision code that triggers a goods movement.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log onto SAP Fiori Launchpad | Log onto the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Usage Decisions (F2345). | The Manage Usage Decisions (F2345) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go: - Plant: 3310 - Material: FG0511 - Inspection Lot Origin: Goods Receipt From Production - Usage Decision Made: No | Alternatively you can search for the <Order ID> or <Batch ID> from previous steps. If Production Order is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Production and choose More Filters. Select Production Order and choose OK. Choose Go. If Batch is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Material and choose More Filters. Select Batch and choose OK. Choose Go |  |
| 4 | Click the Corresponding Inspection Lots | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order: <Order ID> (from previous step). | The Production Order number assigned to Inspection lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Display Defects | Scroll down to the Defects area. Check the details of defects. | Only available if you have recorded a Negative result. For a sensory rejection, there will be a sensory defect, for a relative humidity or rebrand rejection, there will be a tolerance defect displayed. |  |
| 6 | Enable Edit | Choose Edit. In the Document section choose Add, select the path and add the document. | For example, you can upload pictures to visualize the defect. |  |
| 7 | Enter UD code | Go to tab Usage Decision. Depending on the inspection result, select one of the following two options via search help and confirm with Choose. Positive Result UD code: UD04 A1 (Accepted - unrestricted stock) Negative Result UD code: UD04 R1 (Rejected-blocked Stock) UD code: UD04 R320 Rejected–scrap (complete quantity) | If the dialogue Some required characteristics are still open. Do you want to force inspection completion? appears, choose Yes. The corresponding stock posting is processed automatically in background for the complete quantity for Usage Decision codes A1, R1, and R320. |  |
| 8 | Modify Stock Posting (Optional) | In the Inspection Lot Stock section, enter the quantities to be posted to different stock types. | Note If you want to scrap a partial quant with UD code R310, you can split the quantity and post to To Unrestricted Use and To Scrap, respectively. It is also possible to transfer the (partial or full) quantity to a different material if, for example, inspection results show that the product is of higher/lower quality than initially planned in the production order and the higher/lower quality corresponds to different materials.. |  |
| 9 | Save Usage Decision | Choose Save. | The usage decision is made with regard to the finished material. The system automatically triggers the material movements if associated with the usage decision code. Positive Result: The usage decision made for the inspection is Quality Inspection Passed. A goods movement is posted automatically from quality inspection stock to unrestricted use stock for the finished material FG0511. Negative Result: The usage decision made for the inspection is Quality Inspection Failed. A goods movement is posted automatically from quality inspection stock to blocked stock for the finished material FG0511. |  |

### Display Material Stock after Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, we now verify the material movement done based on the usage decision codes by looking again at the material stock.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on | Log on to the SAP Fiori Launchpad as a Warehouse Clerk. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Stock - Single Material (F1076). | The Stock - Single Material (F1076) screen displays. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Enter. Material: FG0511 |  |  |
| 4 | Check Stock Quantity | Check the changed quantities of the Quality Inspection Stock, Unrestricted Use Stock (in case of positive usage decision) or Blocked Stock (in case of negative usage decision). | The stock quantities for finished material FG0511 and <Batch ID> are updated based on the quality inspection and respective usage decision of the previous steps. |  |

## Production Order Processing and Quality Inspection for Packed Roasted Coffee Beans (Dark)

After production of the first component FG0511 (Carton 12 x 1kg Bags Medium Beans Arabica) for the display FG0502 (Display Med + Dark 48 x 1kg Arabica), the second component FG0512 (Carton 12 x 1kg Bags Dark Beans Arabica) needs to be produced as well. The production process for FG0512 (Carton 12 x 1kg Bags Dark Beans Arabica) also comes with full integration into Quality Management.

The production process for FG0512 is quite similar to the one already executed for FG0511. To shorten the process, you can execute Option 1: Create Initial Material Stock for Finished Material FG0512. If you want to execute all production steps for FG0512 as well, use Option 2: Execute Process Steps described for Finished Material FG0511 also for Finished Material FG0512.

### Option 1: Create Initial Material Stock for Packed Roasted Coffee Beans (Dark)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

To shorten the scenario, you can execute an initial stock posting for finished material FG0512.

Note We recommend that you check stock level for material FG0512 first. To do that, log onto the SAP Fiori launchpad as a Production Planner, and open Manage Material Coverage - New Version (F0251A) app. If the stock is sufficient, you can skip this step.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Warehouse Clerk. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Post Goods Movement (MIGO). | The Post Goods Movement (MIGO) screen displays. |  |
| 3 | Choose Document Type | Choose Goods Receipt and Other. Select for Init.entry of stBal. 561 without any stock indicator and press Enter. |  |  |
| 4 | Enter Data | In the Detail Data section, make the following entries and press Enter: On the Where tab: - Goods Movement Type: 561 - Plant: 3310 - Storage Location: 331A On the Quantity tab: - Quantity: <Quantity>, for example, <1000> On the Material tab: - Material: FG0512 On the Batch tab: - Date of Manufacture: <today> Ensure that Item OK is selected. |  |  |
| 5 | Check | Choose Check. | The document is ready to be posted. |  |
| 6 | Post | Choose Post. | Material document XXX is posted and materials are available in stock. |  |

### Option 2: Execute Process Steps described for Packed Roasted Coffee Beans (Medium) also for Packed Dark Roasted Coffee Beans (Dark)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this process step you execute the steps already processed for FG0511 for finished material FG0512.

Prerequisite

A planned order is available for material FG0512.

Procedure

Follow the steps described in the following sections and exchange material FG0511 by material FG0512.

Chapter “Production Order Conversion and Available-to-Promise Check for Packed Roasted Coffee Beans”

- Convert Planned Orders to Production Orders

- Review Material Availability Status for Production Orders

Chapter “Production Order Processing for Packed Roasted Coffee Beans”

- Execute Material Staging

- Release Production Order

- Goods Issue of Batch-Managed Raw Materials via Pick-List

Chapter “Quality Management – Overview Inspection Lots for Packed Roasted Coffee Beans”

- Display Open Inspection Lots

Chapter “Quality Inspection – Within Production (Roasting) for Packed Roasted Coffee Beans"

- Display Routing for Inspection Characteristics

Caution For material FG0512, routings contain material-specific catalogs and limits, which are different to material FG0511

- Display Quality Technician Overview (Optional)

- Display Open Inspection Lots (Optional)

- Execute Quality Inspection with Inspection Points for Operation Roasting

Caution Starting with inspection points with negative results, ending with positive results (only last sample is valid).

- Display Inspection Results History (Optional)

Chapter “Production Order Confirmation of Roasting and Cooling Process for Packed Roasted Coffee Beans”

- Confirm Production for Operation Roasting

- Confirm Production for Operation Cooling

- Monitor Order Progress (Optional)

Chapter “Quality Inspection – Early Goods Receipt Inspection (Before Goods Receipt)”

- Display Inspection Plan for Inspection Characteristics

Caution For material FG0512, the inspection plan contains material-specific catalogs and limits (different to material FG0511)

- Display Quality Technical Overview (Optional)

- Display Open Inspection Lots (Optional)

- Record Inspection Results (Option 1: Before Goods Receipt)

Caution For material FG0512 and for inspection characteristic Rebrand, positive result is 14-17%, negative result is, for example, 21%.

- Display Inspection Results History (Optional)

Chapter “Quality Inspection and Usage Decision – Within Production (Packaging) for Packed Roasted Coffee Beans”

- Record Inspection Results for Operation Packaging

Caution Starting with inspection points with negative results, ending with positive results (only last sample is valid)

- Display Inspection Results History (Optional)

- Display Quality Engineer Overview (Optional)

- Make Usage Decision

Chapter “Production Order Confirmation for Packaging Process of Packed Roasted Coffee Beans”

- Confirm Production of Operation Packaging

- Post Good Receipt for Production Order of Finished Material

- Evaluate Stock/Requirements Situation after Goods Receipt of Finished Material

Chapter”Quality Inspection and Usage Decision – Early Goods Receipt Inspection/ End-Product-Control and Usage Decision (After Goods Receipt)"

- Display Material Stock Before Usage Decision

- Display Open Inspection Lots

- Record Inspection Results (Option 2: After Goods Receipt)

- Display Quality Engineer Overview (Optional)

- Make Usage Decision

- Display Material Stock after Usage Decision

## Production Order Conversion for Coffee Display

After finishing the production of the components FG0511 and FG0512, the production for the coffee display FG0502 can be executed. In addition to the two components produced, a packaging material (PK0508) and a label (PK0504) are needed which should be available in stock. While the two finished materials and the label are batch managed, the packaging material PK0508 (Display Carton Coffee Beans 48 x 1 kg) isn't in direct contact to the coffee, hence not managed in batches and subject to backflush during production process.

The Quality Inspection will be triggered by an inspection lot created during goods receipt of the production order (goods receipt inspection).

Note Even though the display packaging process is often executed in the storage location area, it nevertheless is handled as production step to be able to monitor the packaging times and costs.

### Create and Release Production Order

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to create production orders. Because the order release is executed automatically, production can start immediately.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Planner. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Convert Planned Orders - to Production Orders (CO41) | The Collective Conversion of Planned Orders: Initial screen displays. |  |
| 3 | Enter Restriction | Make the following entries and choose Run Selection: - Planning Plant: 3310 - Production Plant: 3310 - Prodn. Supervisor: YB5 - Order Type: YBM5 - Material: FG0502 | The system displays a list of all planned orders, which match the restriction. |  |
| 4 | Convert Planned Order to Production Order | Select the planned order for conversion for following material, and then choose Convert. Material: FG0502 Note It is possible to change order quantities (such as merging the quantities of 2 planned orders into one order or reducing the quantity of an order). For this scope item reduce the amount of one order to a maximum of 10 PC.. | Planned orders are converted to production orders. Make a note of one production order number ____________________. You may then delete the obsolete planned orders from the selection list before executing order conversion. Run MRP to delete the obsolete planned orders from the database (the planned orders must not be firmed, otherwise MRP would leave them untouched). |  |

## Production Order Processing of Coffee Display

### Goods Issue of Batch-Managed Components

#### Option 1: Goods Issue of Batch-Managed Components via Pick List

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The withdrawal of the raw materials is used for the order. This withdrawal can be done by using the picking list (see production order process for finished materials) or doing a manual goods issue.

Note If the material is set for backflush (in material master or resource), the withdrawing is done automatically after you confirm the operation or the whole order. Backflushing should not be used for batch-managed components (to ensure that the physically picked and consumed batch is the same as the recorded batch).

Prerequisites

Ensure that sufficient material is present on stock. If not enough stock is present, execute step Create Initial Material Stock in the preliminary steps of this scope item.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori Launchpad displays. |  |
| 2 | Access App | Open Pick Components for Production Orders (CO27). | The Picking List: Initial Screen displays. |  |
| 3 | Enter Restriction and Execute | Make the following entries and choose Run: Profile: 000002 Production Order: <Order ID> (from previous steps) Plant: 3310 | The Order Information System: Detail List of Components screen displays. |  |
| 4 | Picking Raw Materials | Select the relevant reservations and choose the upper left Picking. | The Picking list screen displays. On this screen, a list of all the selected reservations is displayed. |  |
| 5 | Batch Determination | Some materials are management by batch (for example FG0511 and FG0512). Select corresponding line item(s) and choose Batch Determination. Click on Copy. | The Batch Determination MM: Select Batches screen displays. |  |
| 6 | Split position into more batches (optional) | Mark one or more lines of the batch managed materials and Choose Split. Make the following entries and choose Copy: Split quantity: <add the split Quantity>. Back in the Picking list screen you can adopt the batch number in the newly created line. | Back to Picking list screen. If multiple line items were selected in step 5, the Picking list screen is displayed after completion of the last line item. |  |
| 7 | Post | Choose Post to post the goods movements. | The goods movements have been posted. |  |

#### Option 2: Manual Goods Issue (Instead of Picking List)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how the system performs a goods issue.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Warehouse Clerk. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Post Goods Movement (MIGO). | The Initial screen displays. |  |
| 3 | Enter Production Order Number | Make the following entries and choose Enter: - Trans./Event: Goods Issue - Reference Document: Order - Order number: <Order ID> (from previous steps) - Movement Type: 261 | The screen name adapts after entries are made. |  |
| 4 | Check Each Item | For each item shown, select Item OK. | Ensure that detail data is expanded. |  |
| 5 | Perform Batch Determination | Choose Stock Determ. for All. | The Batch Determination MM: Select Batches screen displays. |  |
| 6 | Copy Batch Determination Result | Check the required batch and choose Copy. Repeat this step for all the materials you selected. | The Goods Issue Order XXX screen displays. In addition, the batch numbers have been determined. |  |
| 7 | Post Goods Movements | Choose Post. | The goods movements have been posted. |  |

### Review Production Order (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to display and check the production order for finished goods after its release or confirmation.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Supervisor - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Manage Production Operations (F2335). | The Manage Production Operations screen displays. |  |
| 3 | Check Default Area of Responsibility (Supervisor) | Choose the user icon and select App Settings. In the Area of Responsibility dialog box, choose Production Supervisor. Check if only this entry displays, and choose OK: Plant 1 AE 3310 MTS DI- Valuated -Process (YB5). Choose + to select this entry if not displayed; choose the Delete icon to remove any other plant entries and then choose OK. | The Manage Production Operations screen displays. |  |
| 4 | Select Production Order | On the Manage Production Operations screen, choose Adapt Filters to display more filter options. In the Adapt Filters dialog box, choose More Filters. Select Material and Plant and choose OK. Choose Go. Enter the following filters criteria and choose Go. - Material: FG0502 - Plant: 3310 - Status: Released | The Material and Plant fields are then added to filter bar. Selected order displays. |  |
| 5 | Check Detailed Operations Status | Check detailed information for selected production operations. - Operation Issue: - Empty if there is no production order issue. - Delay or Missing Components / PRT issue or Quantity Deviation / Quality Issue information shows if relevant issue exists. - Components: Material, total quantity and issued / open quantity information for order components displays. - Order Schedule: Detailed operation information with relevant work center, confirmed quantity, start and end time displays. - Work Center Schedule: The relevant order covering the same work center capacity (operation is partially confirmed) displays. - Operations not Started: The relevant order that would cover the same work center capacity (operation is released) displays. - Confirmation: Detailed confirmation information with Yield, Scrap, Rework quantity displays if confirmation has been executed. - Inspection: Detailed inspection lots information displays if existing. Note To display the inspection lots, you need further authorizations of SAP_BR_QUALITY_TECHNICIAN (transaction QA03). Ensure that your user has the appropriate authorizations if you want to review the inspection lots. Else this section is empty even though inspection lots are available.. | Detailed operations status displays. Note The Quality Inspection Lot will be created during goods receipt of production order. Hence there is no QM inspection lot visible at that time.. |  |

### Confirm Production Order and Post Goods Receipt

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to confirm the complete production order with all operations and post the Goods Receipt from Production in one step for the Finished Material FG0502.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Operator - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Confirm Production Order (CO15). | The Enter Production Order Confirmation: Initial Screen displays. |  |
| 3 | Enter Production Order Number | Enter the Production Order <Order ID> noted down in previous step and choose Enter. | The Confirmation of Production Order Enter: Actual Data Screen is displayed. |  |
| 4 | Check Confirmation Type | Review the confirmation type. Per defauIt Final Confirmation and Clear Open Reservations is flagged. If you want to execute only partial confirmation change radio button and unflag Clear Open Reservations. |  |  |
| 5 | Enter Fields in Actual Data Section | In the Actual Data section, make the following entries: - Yield Quantity : <enter the amount you want to Confirm> Scrap: <enter the amount you want to Scrap> |  |  |
| 6 | Enter Production Date | Choose Goods Movements Overview button. A pop up is displayed. Due to the fact that the finished product has an expiration date (which will be calculated according to the production date you enter) make the following entries and choose OK. Expiry date/Manu.date: <today´s date> | The finished product has an expiration date which will be calculated according to the production date you entered during goods receipt. |  |
| 7 | Enter Batch for Finished Material | On the Confirmation of Production Order Enter: Goods Movements screen make the following entries: Batch: <manually enter a new batch number> (for FG0502 manually enter a new batch number, for PK0504 select an appropriate one) | Note If you have already done a partial confirmation before, choose the batch number already created during first partial confirmation for FG0502 from the drop-down menu so that only one batch number is available per production order.. Note down the batch number ___________________. |  |
| 8 | Save your Entries | Choose Post to execute the partial respective final confirmation for this order. | Confirmation has been created. Note If you have executed only a partial confirmation you can repeat steps 1 to 7 with the settings for partial or final confirmation.. |  |

If customer complaints about product quality are raised, these complaints are normally either raised with respect to a batch number or the expiration date. To figure out the expiration date for the batch created during goods receipt, access the app Display Batch Changes, enter material FG0502, batch number created in this step and the plant 3310. Choose enter and press the button Display in the top section and afterwards the tab Basic Data 1. Note down the SLED/BBD for the corresponding batch _____________________. You will use the SLED/BBD date as an option to execute batch analysis later on in this test script.

Result

With goods receipt for production order, an inspection lot is created for material FG0502. With this inspection lot the End-Product Control is performed with 100 % visual check of each packed and labeled display.

## Quality Inspection – Goods Receipt Inspection and Usage Decision for Coffee Display

### Display Material Stock Before Quality Inspection (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, a review of the finished material stock is done. Due to quality inspection, the material is not available for other processing.

Prerequisite

The goods receipt from production order was posted for the finished material directly to quality inspection stock. This is caused by material master settings for inspection type 04 Goods Receipt Inspection from Production.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as an Inventory Manager. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Stock - Single Material (F1076). | The Stock - Single Material (F1076) screen displays. |  |
| 3 | Enter Material | Make the following entries and press Enter. Material: FG0502 | Stock for the material is displayed. |  |
| 4 | Check Stock Quantity | Check the Quality Inspection Stock under the plant 3310 and batch <Batch ID> from previous step. | Stock for the material, batch and storage location is checked. |  |
| 5 | Check Inspection Lots | Choose Manage Stock on the bottom of the screen. Then choose Manage Inspection Lots on the bottom of the screen. Note Ensure your user has necessary authorization to view Manage Inspection Lots.. | The Inspection Lots for the selected material are displayed. |  |

### Display Inspection Plan for Inspection Characteristics

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, you display the inspection plan that is related to the finished goods. The inspection plan defines all inspection characteristics used later in the quality inspection.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Display Inspection Plan (QP03). | The Display Inspection Plan: Initial Screen displays. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Continue. Material: FG0502 Plant: 3310 |  |  |
| 4 | Go to Inspection Characteristics Overview Screen | Choose Inspection Plan with Group Counter GrC 1 and choose Operations. Select the entry for Operation 0010 and choose Inspection Characteristics. Note If you could not find Inspection Characteristics in your screen, choose Menu > Goto > Inspection Characteristic Overview .. | The Display Inspection Plan: Characteristic Overview screen displays. |  |
| 5 | Review the Inspection Characteristics | There is one Master Inspection Characteristics: it is Qualitative – Visual Pack Check. Sample size is 100% to be inspected. Auto-defect recording is defined if charac. is rejected. | You have now reviewed the inspection characteristics for which data has to be maintained later. |  |

### Display Quality Technician Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this optional step, you can display an overview of open inspection lots via the overview page for quality technicians.

Prerequisite

The goods receipt from production order was posted for the finished material directly to quality inspection stock.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Technician Overview (F2361). | The Quality Technician Overview (F2361) screen displays. You can get an overview of the inspection lots with open results recording and the inspection lots without an inspection plan. Choose card Inspection Lots with Open Results Recording, No Inspection Points if you want to record results for the goods receipt inspection lot. |  |

### Display Open Inspection Lots

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Inspection Lots (F2343). | The Manage Inspection Lots (F2343) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go. - Plant: 3310 - Material: FG0502 Inspection Lot Status: Outstanding | Alternatively, you can search for the batch <Batch ID> (from previous step). |  |
| 4 | Select the Corresponding Inspection Lot | In the inspection lot list, select the inspection lot with column Production Order equals to the Production Order : <Order ID> (from previous step). | The Production Order number assigned with Inspection lot is displayed. If the Production Order column does not appear, please choose the button Settings. In the View Settings dialog box, select Production Order and choose OK. |  |
| 5 | Check Inspection Lot | In the inspection lot list you can watch the progress of characteristics. | Make a note of the inspection lot: Inspection Lot ______________ with inspection type 04 (Goods Receipt from Production). Progress is 0%. |  |
| 6 | Check Inspection Lot | In the Origin tab, verify the referenced production order and material document. | The production order and material document from which the inspection lot originates are displayed. |  |
| 7 | Check Inspection Lot Quantities | In the Origin tab, check the Lot Quantities, Sample Size and Postings. | The size of the sample for quality inspection is displayed (100% of inspection lot quantity). Actual Lot Quantity and Postings are according to goods receipt quantity from previous step. |  |
| 8 | Check Material Document | In the Stock Postings tab, check details of the assigned material document, which has created the inspection lot. | Movement Type of this material document is 101 GR goods receipt. Quantity is as posted in the previous step. |  |

### Record Inspection Results (Hierarchical Worklist)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, the quality technician performs the quality inspection and records the inspection result(s) for operation 0010 (Display Packaging Check). The result is recorded in the inspection lot according to the defined inspection parameters in the inspection plan.

Prerequisite

The inspection is processed based on inspection lot. In this scope item, the inspection lot is automatically created upon goods receipt for production order.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Record Inspection Results - Hierarchical Worklist (QE51N). | The Results Recording Worklist screen displays. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Execute. Plant： 3310 Insp. Lot Origin： 04 (Goods Receipt from Production) Material: FG0502 Batch: <Batch ID> from previous step Lot Created: <choose one week ago until today>> | The Results Recording Worklist Screen displays. Caution In Cloud system, Dynamic Selection (e.g. for production order) is not offered. Alternatively you can search for the <Batch ID> or <Inspection Lot ID> from previous step.. |  |
| 4 | Select the Corresponding Inspection Lot | On the left pane, expand the corresponding inspection lot which you want to record a result. | If a dialogue appears asking to save, choose Cancel. |  |
| 5 | Choose the Characteristic | Select the characteristic which is under the activity 0010 Display Pack-Control Work Center Quality Check Point with a click (or use icon Select a Characteristic). | When the characteristic is selected, Record Results for XXXX: Characteristic Single Screen, opens where the class size can be recorded. |  |
| 6 | Enter Inspection Result for Characteristic | Depending on the inspection result, classify the no. of inspected displays and then confirm with Save. - Positive Result - Visual Pack Check: <enter no. of displays in class size> of class OK - Negative Result - Visual Pack Check: <enter no. of displays in class size> of class NOK | For the qualitative characteristic Visual Check a 100% check is performed, assigning the no. of displays to the corresponding class OK or NOK. Auto Valuation is done by the system. The status is set to 5 Processing is completed. |  |

### Display Quality Engineer Overview (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection is done and the inspection result is recorded. You must decide whether or not to accept the inspection lot depending on the inspection result. As an optional entry point for making this usage decision, you can use the app Quality Engineer Overview (F2360). Furthermore, the Quality Engineer Overview (F2360) app displays inspection lots without an inspection plan (e.g. if automatic assignment of an inspection plan is not set up in the material master or if automatic assignment failed), inspection lots without usage decision and inspection lots ready for usage decision. If negative inspection results are recorded, you can check the defect details in app Manage Defects (F2649) directed from the app Quality Engineer Overview (F2360) via the cards Inspection Lots with Defects, Top Defective Materials, or Top Defects.

Caution For this scope item example, auto-defect recording is performed in quality inspection for goods receipt (End-Product Control) because deviations shall be tracked as defects later.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Quality Engineer Overview (F2360). | The Quality Engineer Overview (F2360) screen displays. |  |
| 3 | In Card: Inspection Lots without Inspection Plan choose one entry (optional) |  | Assign missing data and ensure automatic inspection plan determination for further usage. |  |
| 4a | In Card: Select Inspection Lots Without Usage Decision (Optional) | Choose card Inspection Lots Without Usage Decision. | You can process further as described in Make Usage Decision. |  |
| 4b | Select the Corresponding Inspection Lot you have created before (optional) | In the inspection lot list, select the inspection lot. | The Usage Decisions screen displays. Alternative entry to Make Usage Decision. |  |
| 4c | Select one of the Inspection Lot by Results Recording Status (Optional) | Select one of the Inspection Lot by Results Recording Status (Optional). | Alternative entry to Make Usage Decision. |  |
| 5 | Select Card Inspection Lots Ready for Usage Decision (optional) |  | All inspection lots with positive results get collectively an accepted usage decision (e.g. defaulted by A1) when pressing button Make Quick Usage Decision. |  |
| 6 | In Card: Inspection Lots with Defects (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the inspection lot origins, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 7 | In Card: Top Defective Materials (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the materials inspected, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. | The defect details are displayed. |  |
| 8 | In Card: Top Defects (optional) | (In case negative inspection results have been recorded and auto-defect recording has been defined). Select one of the defect codes recorded, then you will be directed to app Manage Defects (F2649) automatically. Choose the relevant defect ID to check the details of the defect. |  |  |

### Make Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

The inspection is done and the inspection result is recorded. You must decide whether or not to accept the finished material depending on the inspection result. If you do not want to use the Quality Engineer Overview (F2360), you can open the Manage Usage Decisions (F2345) app directly.

In order to close the inspection lot, the quality engineer makes a usage decision. As the inspection lot of origin 04 is stock-relevant, there is a follow-up action assigned to the usage decision code that triggers a goods movement.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on to SAP Fiori Launchpad | Log on to the SAP Fiori Launchpad as a Quality Engineer. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Manage Usage Decisions (F2345). | The Manage Usage Decisions (F2345) screen displays. |  |
| 3 | Enter Filter Fields | Make the following entries and choose Go: - Plant: 3310 - Material: FG0502 - Usage Decision Made: No | Alternatively you can search for the <Order ID> or <Batch ID> from previous steps. If Production Order is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Production and choose More Filters. Select Production Order and choose OK. Choose Go. If Batch is not displayed in the Filters, choose Adapt Filters. On the popup screen Adapt Filters, find Material and choose More Filters. Select Batch and choose OK. Choose Go. |  |
| 4 | Select the Corresponding Inspection Lot | In the inspection lot list, select the inspection lot. |  |  |
| 5 | Display Defects | Scroll down to the Defects area. Check the details of Defects. | Only available if you have recorded a Negative result. For a visual check rejection, there will be a packaging defect. |  |
| 6 | Enable Edit | Choose Edit. In the Document section choose Upload, select the path and add the document. | For example, you can upload pictures to visualize the defect. |  |
| 7 | Enter UD code | Depending on the inspection result, select one of the following two options via search help and confirm with Choose. Positive Result UD code: UD04 A1 (Accepted - unrestricted stock) Negative Result UD code: UD04 R1 (Rejected-blocked Stock) UD code: UD04 R320 Rejected–scrap (complete quantity) | If the dialogue 'Some required characteristics are still open. Do you want to force inspection completion?' appears, choose Yes. The corresponding stock posting is processed automatically in background for the complete quantity for Usage Decision codes A1, R1, and R320. Note If you want to run a rework process for material FG0502 due to failed quality inspection, you can follow the test script 6A9 (Rework with Quality Management – Consumer Products). You need to follow all test steps described in the section “Rework Production Order of Stock Manufactured Material with Quality Management”.. |  |
| 8 | Modify Stock Posting (Optional) | In the Inspection Lot Stock section, enter the quantities to be posted to different stock types. | Note If you want to scrap a partial quantity with UD code R310, you can split the quantity and post to To Unrestricted Use and To Scrap, respectively. It is also possible to transfer the (partial or full) quantity to a different material if, for example, inspection results show that the product is of higher/lower quality than initially planned in the production order and the higher/lower quality corresponds to different materials.. |  |
| 9 | Save Usage Decision | Choose Save. | The usage decision is made with regard to the manufactured goods. The system automatically triggers the material movements if associated with the usage decision code. Positive Result: The usage decision made for the inspection is Quality Inspection Passed. A goods movement is posted automatically from quality inspection stock to unrestricted use stock for the finished material FG0502. Negative Result: The usage decision made for the inspection is Quality Inspection Failed. A goods movement is posted automatically from quality inspection stock to blocked stock for the finished material FG0502. |  |

### Display Material Stock After Usage Decision

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

In this step, we now verify the material movement done based on the usage decision codes by looking again at the material stock.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log on | Log on to the SAP Fiori Launchpad as a Warehouse Clerk. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Stock - Single Material (F1076). | The Stock - Single Material (F1076) screen displays. |  |
| 3 | Enter Selection Fields | Make the following entries and choose Enter. Material: FG0502 |  |  |
| 4 | Check Stock Quantity | Check the changed quantities of the Quality Inspection Stock, Unrestricted Use Stock (in case of positive usage decision) or Blocked Stock (in case of negative usage decision). | The stock quantities for finished material FG0502 and <Batch ID> are updated based on the quality inspection and respective usage decision of the previous steps. |  |

## Evaluations

### Review Scrap Report (Optional)

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

This process step shows you how to review the scrap report.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log on to the SAP Fiori launchpad as a Production Supervisor - Discrete Manufacturing. | The SAP Fiori launchpad displays. |  |
| 2 | Access the App | Open Material Scrap - Maximum Deviation (F2035). | The Material Scrap screen displays. |  |
| 3 | Investigate Scrap | Materials Scrap can be displayed by various filters. You could choose the corresponding material in Chart to display the details. |  |  |

### Batch Information Cockpit - Top-Down and Bottom-Up Analysis

Test Administration

Customer project: Fill in the project-specific parts.

| Test Case ID | <X.XX> | Testing Date: |  |
| --- | --- | --- | --- |
| Tester Name: |  | Duration: |  |
| Business Role(s): |  | Responsibility: | <State the Service Provider, Customer or Joint Service Provider and Customer> |

Purpose

With this functionality you can identify which batches are delivered to which customer and which raw material batches are used for the production of which finished product. You can adapt the selection in the Batch Information Cockpit to customer-specific requirements by creating user-specific groups. You can get additional information by using subsequent actions.

In this case the evaluation is limited to the production process. As soon as you have purchased the raw materials and/or delivered the finished good FG0502 to a customer the evaluation is automatically enhanced with these information.

Procedure

| Test Step # | Test Step Name | Instruction | Expected Result | Pass / Fail / Comment |
| --- | --- | --- | --- | --- |
| 1 | Log On | Log onto the SAP Fiori Launchpad as a Quality Technician. | The SAP Fiori Launchpad displays. |  |
| 2 | Access the App | Open Batch Information Cockpit (BMBC). | The Batch Information Cockpit screen displays. |  |
| 3 | Select Batches | On the Batch Information Cockpit screen, make the following entries. Go to the Material tab. Material : FG0502 Plant: 3310 Go to the Classification tab. Batch Class: YB_EXP_DATE_001 Or Go to Search with Characteristics area. Selection Class: YB_EXP_DATE_001 Choose Create values. Enter Characteristics value. e.g. <SLED/BBD from previous step> Note For batch class and characteristics relevant materials, refer to Master Data, Organizational Data, and Other Data.. Choose Execute Selection. | One or multiple batches are displayed in the left screen area Selection Result for Batches. The standard sorting is done per material number. |  |
| 4 | Expand Selection | In the section Selection Result for Batches. Select the checkbox for FG0502 and then choose Expand subtree or Expand node. | The selection result is analyzed and a batch for further investigation is identified. |  |
| 5 | Start Analysis | Right-click on one selected batch and choose either Usage(Top-Down) or Usage(Bottom-up). | The batch where-used list displays. |  |
| 6 | Analyze the Batch Where Used List | The usage of the batch can be analyzed and the specific documents (such as batch master record, production orders and sales order) can be accessed via double click. |  |  |
| 7 | Analyze Assigned Inspection Lots | Mark one line in the Details (Usage) table and choose button Inspection Lots to review inspection lots assigned to the selected element. |  |  |
| 8 | Analyze Material Document | Choose Details again and select Usage. Select one document and choose Documents. | The usage displays again and after selecting documents the respective material document display. |  |
| 9 | Go to the Material Document | Choose the material document in the pop up dialog and choose Choose (F2). To display the material document, you need further authorizations that are granted by your company, for example, SAP_BR_INVENTORY_MANAGER. | The material document displays. Note that your user has to have the appropriate authorizations. |  |

# Appendix

## Process Integration

N/A

## Preceding Processes

The following processes and conditions can precede this process.

| Process | Business Condition |
| --- | --- |
| 5OI - Capacity Planning and Leveling - Consumer Products | You can execute this scope item before executing 5HK. In that case you can skip the process steps "Create Planned Independent Requirements" and "Materials Requirements Planning at Plant Level" of this test script. |

## Succeeding Processes

After completing the activities in this test script, you can continue testing the following business processes:

| Process | Business Condition |
| --- | --- |
| Technical Completion of Production Order (Optional) | You can access Schedule Order Technical Completion Run (F3841) app as a Production Supervisor - Discrete Manufacturing to schedule a job to set production orders as Technical Completion. If you need to set a production order as Technical Completion separately, you can access Manage Material Coverage - New Version (F0251A) app as a Production Planner, find your production order and select Delete. |
| 3F0 - Event-Based Production Cost Posting | This procedure updates work-in-process calculation, overhead calculation, and variance calculation by linking the posting to the events Note Start from CE2105, the default Best Practice content is based on event-based result analysis for new customer install and add new country only, rather than previous period-end posting. For existing customers (who activated content before 2105), the default remain unchanged to Period-End Closing – Plant (BEI) till customer makes the decision to switch. For migration to Event based from Periodic please refer Production Settlement in Parallel Ledgers. If you are still using period-end posting, you may need to refer to non-standard Scope Item BEI - Period-End Closing – Plant.. |
| BEI - Period-End Closing – Plant | Use this scope item to execute period-end closing. |
| Close Confirmed Production/Process Orders (Optional) | Search for Schedule Order Closing Runs - Close Production/Process Orders (F4305) app as a Production Supervisor - Discrete Manufacturing to schedule a job to set orders as Closed (CLSD). |
| BMC - Core Inventory Management | Complete the following activities: Perform the selected step you want to perform for the Blocked stock. |
| 5HL - Sell from Stock – Consumer Products | You can follow the test script to sell the produced Packed Coffee Beans and Displays from this scope items to the customer. |
| 2V0 - SAP Fiori Analytical Apps for Quality Management | You can follow the test script to analyze the defects and accepted/rejected inspection lots and characteristics in relevant timelines |

Typographic Conventions

| Type Style | Description |
| --- | --- |
| Example | Words or characters quoted from the screen. These include field names, screen titles, pushbuttons labels, menu names, menu paths, and menu options. Textual cross-references to other documents. |
| Example | Emphasized words or expressions. |
| EXAMPLE | Technical names of system objects. These include report names, program names, transaction codes, table names, and key concepts of a programming language when they are surrounded by body text, for example, SELECT and INCLUDE. |
| Example | Output on the screen. This includes file and directory names and their paths, messages, names of variables and parameters, source text, and names of installation, upgrade and database tools. |
| Example | Exact user entry. These are words or characters that you enter in the system exactly as they appear in the documentation. |
| <Example> | Variable user entry. Angle brackets indicate that you replace these words and characters with appropriate entries to make entries in the system. |
| EXAMPLE | Keys on the keyboard, for example, F2 or ENTER. |

www.sap.com/contactsap

© 2026 SAP SE or an SAP affiliate company. All rights reserved. No part of this publication may be reproduced or transmitted in any form or for any purpose without the express permission of SAP SE or an SAP affiliate company. The information contained herein may be changed without prior notice. Some software products marketed by SAP SE and its distributors contain proprietary software components of other software vendors. National product specifications may vary. These materials are provided by SAP SE or an SAP affiliate company for informational purposes only, without representation or warranty of any kind, and SAP or its affiliated companies shall not be liable for errors or omissions with respect to the materials. The only warranties for SAP or SAP affiliate company products and services are those that are set forth in the express warranty statements accompanying such products and services, if any. Nothing herein should be construed as constituting an additional warranty. SAP and other SAP products and services mentioned herein as well as their respective logos are trademarks or registered trademarks of SAP SE (or an SAP affiliate company) in Germany and other countries. All other product and service names mentioned are the trademarks of their respective companies. See www.sap.com/copyright for additional trademark information and notices.